-------------------------------------------------------------------------

PCF554.PRG Seek Rate Adjustment Program for the PCF554

1988 Atari Corporation

Date of release: January 11,1989

-------------------------------------------------------------------------

PCF554.PRG allows you to use the Atari 5 1/4" disk drive (known as

the Atari PCF554) with your MEGA or ST.  It works with all versions of

TOS, including the forthcoming TOS 1.4.

To use it, copy PCF554.PRG to the AUTO folder of your boot disk. If

you boot from floppy, and do not have an AUTO folder, see your MEGA or

ST owner's manual for information on how to create a folder named AUTO

on your boot disk.  Then copy PCF554.PRG to the newly created folder.

If you have a hard disk, copy PCF554.PRG into your C:\AUTO folder.

Attach the PCF554 disk drive to the floppy port on your MEGA or ST

computer system.

Ensure that the PCF554 drive is connected to be the second floppy

drive (B:) in your system. To do this, make sure that the disk drive

cord running from your computer to your PCF554 is connected to the IN

port on the back of the disk drive. Please remember that the disk drive

cord must be plugged into the IN port on the back of the computer or

the computer will not recognize your disk drive.

Power on your computer system, and PCF554.PRG will run automatically

from your AUTO folder, and display an informative message. The message

will read as follows:

PCF554 Driver

Copyright 1988, Atari Corp.

6ms seek rate set on B:

You should then be able to use the PCF554 disk drive normally.

PCF554.PRG works by setting the seek rate (that is, the speed with

which the drive accesses information on the disk) on drive B to six (6)

milliseconds.  You can use a 3 1/2" disk drive as drive B while using

PCF554.PRG, but it will be slower than normal.

In order to restore the seek rate to make the 3 1/2" drive run at

normal speed, you must remove PCF554.PRG from your AUTO folder or

rename it to PCF554.PRX. In either case, the computer will not execute

the program next time you boot your system. Then turn off your MEGA or

ST computer system, hook up the 3 1/2" drive, and boot normally. Do NOT

attempt to connect or  disconnect disk drives while your MEGA or ST

computer system is turned on.

For those using the PCF554 disk drive to access 5 1/4 MS-DOS media,

the PCF554 will read and write to MS-DOS diskettes without modification

as long as you are using the PCF554.PRG program. However if you format

a disk on your ST from the desktop, the format that TOS generates is

not readable by an MS-DOS compatible machine. To format a disk so that

you can use it on both machines, you will need to use a program that is

capable of formatting a disk for use on MS-DOS. There are several such

programs in the public domain and available from User Groups or you can

format your diskettes on a PC or compatible. (The upcoming TOS 1.4

generates a format that can be read by MS-DOS machines directly.)

If you have any questions or comments about PCF554.PRG, please

contact Atari's Software Support personnel at (408) 745-2004 between

8AM and 5PM Monday - Friday Pacific Time.