Programmer's Manual for access to the TapeBIOS software driver Version 1.5

==========================================================================

TAPEBIOS.TOS is a program that provides the low-level BIOS routines

to access a SCSI tape drive connected the the Atari ST's DMA bus and also

the Atari TT's DMA bus(not the SCSI bus). The low-level routines are

documented in this section and all defines are provided for the GNU 'C'

compiler through tapebind.h and tapebind.cpp.

Any calls to the tape BIOS need to be made through the trap #10

vector with command $ACE, if this is not met then the command is then

passed to the next vector in the XBRA chain, usually displaying a full line

of the familiar bombs as nothing uses trap #10 as standard in the Atari

Operating System.

Ah yes, When TAPEBIOS.TOS installs itself, it hooks itself into the OS

with the XBRA ID of 'TAPE' into the trap #10 chain, and also installs itself

into the COOKIE JAR, again with the ID of 'TAPE' with the vector containing

the version number of the TapeBIOS software. The driver remains resident after

loading with approximately 2KB taken from the system.

The Commands

============

Most commands documented below comply to the standard SCSI command

set, although some commands are vendor specific and need to noted if any

of these commands are going to be used over a variety of vendor specific

tape drives.

Definitions taken from TAPEBIND.H

---------------------------------

/* DEFINITIONS FOR TAPE I/O FUNCTIONS */

#define Tfunc	0x0ACE	/* BIOS Call for Tape function */

#define Ready	0	/* Tape Ready */

#define Read	1	/* Tape Read */

#define Write	2	/* Tape Write */

#define Rewind	3	/* Tape Rewind */

#define Load	4	/* Tape Load */

#define Unload	5	/* Tape Unload */

#define WrtFM	6	/* Write FileMark */

#define ReqSen	7	/* Request Sense */

#define ModSel	8	/* Mode Select */

#define ModSen	9	/* Mode Sense */

#define Space	10	/* Tape Space */

#define Erase	11	/* Tape Erase */

#define Inquire 12	/* Tape Inquiry */

#define Tready()	trap_10_wwll(Tfunc, Ready, 0L, 0L)

#define Tread(a, b)	trap_10_wwll(Tfunc, Read, a, b)

#define Twrite(a, b)	trap_10_wwll(Tfunc, Write, a, b)

#define Trewind()	trap_10_wwll(Tfunc, Rewind, 0L, 0L)

#define Tload(a)	trap_10_wwll(Tfunc, Load, 0L, a)

#define Tunload(a)	trap_10_wwll(Tfunc, Unload, 0L, a)

#define Tfmark(a)	trap_10_wwll(Tfunc, WrtFM, 0L, a)

#define Trsense(a, b)	trap_10_wwll(Tfunc, ReqSen, a, b)

#define Tmselect(a, b)	trap_10_wwll(Tfunc, ModSel, a, b)

#define Tmsense(a, b)	trap_10_wwll(Tfunc, ModSen, a, b)

#define Tspace(a)	trap_10_wwll(Tfunc, Space, 0L, a)

#define Terase()	trap_10_wwll(Tfunc, Erase, 0L, 0L)

#define Tinquir(a, b)	trap_10_wwll(Tfunc, Inquire, a, b)

NOTE: When using any of the above commands which support the fourth parameter

or Number of Blocks, as it is commonly known, DO NOT use a value greater than

65535 unless you know exactly what you are doing. If this value is increased

over this limit then the timeout code will fail within the driver. If you

need to have more than 65535 blocks for your program to work then split the

command over two new commands.

For example, in 'C'.

status = Tspace(80000L);  - This will fail.

Use this instead.

status = Tspace(40000L);  }  This will

status = Tspace(40000L);  }  work fine.

If you don't like the above approach the 'swait:' timeout value can be increased

if necessary, although this will lead to longer delays when something wrong

occurs. I don't advise increasing the 'swait:' value and suggest you do use

the above approach.

Status Codes

------------

A single byte of status information is returned to the initiator

after the command is completed. The status code informs the initiator whether

the command was executed, and whether errors occured in execution.

The status is not sent if the command is reset or aborted by a message

passed from the initiator.

Status byte Code Values:

HEX		Status represented

---		------------------

00		GOOD

02		CHECK CONDITION

08		BUSY

10		INTERMEDIATE/GOOD

18		RESERVATION CONFLICT

GOOD: This status indicates that the TAPE CONTROLLER has successfully

completed the command.

CHECK CONDITION: This status is set when any error, or abnormal condition

that causes sense data to be set occurs. The REQUEST SENSE command should

be used to establish the precise nature of the condition.

BUSY: This status is returned to the initiator when the TAPE CONTROLLER

is already processing a command, and cannot accept the command supplied.

INTERMEDIATE: This status is identical to the GOOD status except that it is

sent in response to a Linked command. If this status is sent the TAPE

CONTROLLER will send a LINK COMPLETE message (with or without FLAG) and

then request the next command without first disconnecting. If a Linked

command does not end with an INTERMEDIATE status, the linked chain is broken.

RESERVATION CONFLICT: This status indicates that the TAPE CONTROLLER has been

reserved for another initiator, and cannot therefore accept the command.

Test Unit Ready

---------------

Assembler:	tapebios(0xACE, Ready, 0L, 0L)

'C'	 :	Tready();

The Test Unit Ready command provides a means to check if the Tape

drive is ready. This is not a request for self test.

If the Drive is ready, the command is terminated with GOOD status.

Read

----

Assembler:	tapebios(0xACE, Read, (long)(BUFADR), (long)(blocks))

'C'	 :	Tread((long)(BUFADR), (long)(blocks));

The Read command transfers one or more blocks to the initiator

beginning with the next block on the tape. The Fixed bit specifies both the

meaning of the transfer length field and whether the fixed-length or variable-

length block(s) are to be transferred. The TAPE CONTROLLER is operating in

Variable Block Mode if the Block Length Bytes in the Mode Select Command are

set to zero, otherwise the TAPE CONTROLLER is in Fixed mode. The Fixed bit

must match the current transfer mode of the TAPE CONTROLLER. If the Fixed

Bit does no match the current mode the TAPE CONTROLLER will reject the command

by returning CHECK CONDITION status and by setting the Sense Key to ILLEGAL

REQUEST.

Variable Mode Operation.

If the Fixed bit is zero, a single block is transferred, with the

Transfer Length specifying the maximum number of bytes the initiator has

allocated for the returned data. This form of the Read command is valid

only when the TAPE CONTROLLER is in the variable block mode.

Several possibilities exist:-

- The data block read from the tape is exactly the same length as the

transfer length. All the data in the block is transferred and the command

completes with a GOOD status.

- The data block read from tape is shorter than the transfer length.

All the data in the block is transferred and the command completes with CHECK

CONDITION status. The ILI and VALID bits are set in the sense information.

The information bytes will be set to the difference between the Transfer

Length and the data block length.

- The data block read from tape is longer than the transfer length.

The Transfer Length value is used to determine how many bytes should be

transferred to the initiator, the remaining bytes being discarded. The command

completes with a CHECK CONDITION status. The ILI and VALID bits are set in

the sense information. The Information bytes will be set to the difference

between the Transfer Length and the data block length. This will be a 2's

complement negative value.

- The data block read from tape is longer than the available buffer

within the TAPE CONTROLLER. The Transfer Length value is used to determine

how many bytes should be transferred to the initiator, the remaining bytes

being discarded. The command completes with CHECK CONDITION status. Th ILI

bit will be set in the sense information, the information bytes will be set

to zero.

Fixed Mode Operation.

If the Fixed bit is one, the Transfer Length specifies the number of

blocks to be transferred to the initiator. This form of the Read Command is

valid only if the TAPE CONTROLLER is currently operating in Fixed Bit mode.

The Current Block Length is the Block Length defined in the Mode Select

command.

A successful Read Command with the Fixed Bit set will transfer the

number of blocks specified by the Transfer Length bytes to the initiator.

Each block will be the same length as that specified by the MODE SET command.

Upon termination of the Read Command, the tape will be positioned after the

last block transferred (EOT side).

If the Fixed bit is one and if a block is read which is larger or

smaller than the Current Block Length, CHECK CONDITION status is returned

to the initiator. The ILI bit and the Valid bit in Extended Sense will be

set to one. The Information Bytes will be set to the difference (residue)

between the requested Transfer Length and the actual number of blocks read

(not including the incorrect length block). Upon termination, the tape will

be positioned after the incorrect length block (EOT side).

Filemark Detection.

If the TAPE CONTROLLER reads a filemark during a Read Command, it will

send a CHECK CONDITION status to the initiator and will set the Filemark bit

in the Extended Sense. Upon termination, the tape will be positioned after

the filemark (EOT side). If the Fixed bit is one, the TAPE CONTROLLER will set

the Valid bit to one and the Information Bytes will be set to the difference

(residue) between the requested Transfer Length and the actual number of blocks

read (not including the filemark).

End of Tape Detection.

If a Logical Unit encounters the EOT marker during a Read command, the

TAPE CONTROLLER returns a CHECK CONDITION status to the initiator and sets

the End of Medium (EOM) bit to one in the Sense bytes. If the Fixed bit is one,

the TAPE CONTROLLER sets the Valid bit to one and the Information Bytes to

the difference (residue) between the requested Transfer Length and the actual

number of blocks successfully read. The tape will be positioned after the last

block transferred.

It is possible to read data after the EOT marker, but in this case all

transfers will terminate after a single block with the EOM bit set as above.

Great care should be taken when commanding the TAPE CONTROLLER to read past

EOT since it may then be possible to run the tape off the supply reel. In an

attempt to avoid this situation the TAPE CONTROLLER always selects slow speed

(where available) during post-EOT reads, and "no-data" detection occurs sooner

than normal.

Note that the TAPE CONTROLLER maintains a look-ahead buffer, which may

contain upto 64 blocks, therefore the tape will not normally move for each

block transferred to the initiator.

When the Transfer Length is zero, no data will be transferred and the

tape position on the drive will not be changed. This condition is not

considered as an error.

Write

-----

Assembler:	tapebios(0xACE, Write, (long)(BUFADR), (long)(blocks))

'C'	 :	Twrite((long)(BUFADR), (long)(blocks));

The Write command transfers one or more blocks from the initiator to

the current position on the Logical Unit. The Fixed bit specified both the

meaning of the Transfer Length field and whether fixed-length or variable-

length block(s) are to be transferred. The TAPE CONTROLLER is operating in

Variable Block Mode if the Block Length Bytes in the Mode Select Command are

set to zero, otherwise the TAPE CONTROLLER is in Fixed Mode. The Fixed bit

must match the current transfer mode of the TAPE CONTROLLER. If the Fixed bit

does not match the current mode the TAPE CONTROLLER will reject the command

by returning CHECK CONDITION status and by setting the Sense Key to ILLEGAL

REQUEST.

Variable Mode.

If the Fixed bit is zero, a single block will be transferred from the

initiator and will be written to the tape beginning at the current tape

position. The Transfer Length specifies the length of the block to be written

(in bytes). The requested block length must be within the Minimum and Maximum

Block Length range. If this condition is not met, a CHECK CONDITION status is

returned with the Sense Key set to ILLEGAL REQUEST and no data is written. Upon

successful termination, the tape is positioned after the block written by this

command (End-of-Tape side).

Fixed Mode.

If the Fixed bit is one, the Transfer Length field specifies the number

of block(s) to be transferred to the tape beginning at the current tape

position. This form of the Write command is valid only if the TAPE CONTROLLER

is currently operating in Fixed Block Mode (see the Read command description).

Upon termination, the tape will be positioned after the block(s) written by

this command (End-of-Tape side).

End of Tape Detection.

If the early warning End-of-Tape marker is encountered while writing,

an attempt to finish writing any buffered data is made (unless the PEOT bit is

set - see MODE SELECT command). The command will terminate with CHECK CONDITION

status and the EOM bit in Extended Sense will be set to one. If the Fixed bit

is one and the TAPE CONTROLLER is not buffered (Buffered Mode of the Mode Sense

command is zero), then the valid bit in Extended Sense will be set to one and

the Information Bytes will be set to the difference (residue) between the

requested Transfer Length and the actual number of blocks written to the tape.

If the Fixed bit is one and the TAPE CONTROLLER is buffered (Buffered Mode of

the Mode Sense command is one), then the valid bit will be set to one and the

Information Bytes will be set to the total number of blocks not written (the

number of blocks not transferred from the initiator plus the number of blocks

remaining in the Target's buffer). Note that in this case it is possible for

the value in the Information Bytes to exceed the Transfer Length.

Note that the TAPE CONTROLLER has a large buffer area, which is capable

of holding up to 64 blocks. Therefore in buffered mode the tape position may

not change with each write command. The TAPE CONTROLLER will write all data

contained in the buffer on receipt of a write filemark, space, or rewind

command.

When the Transfer Length is zero, no data will be transferred and the

current tape position on the Tape Drive will not change. This condition is not

considered an error.

Rewind

------

Assembler:	tapebios(0xACE, Rewind, 0L, 0L)

'C'	 :	Trewind();

The Rewind command requests that the Target rewind the Logical

Unit to the Beginning-of-Medium or Load-Point.

The immediate bit, If set, indicates that the operation will complete

by returning the status byte as soon as the rewind is started. The immed bit,

If zero, indicates that the completion status will be returned only after the

rewind is completed.

If a Rewind command is issued to the TAPE CONTROLLER when the tape is

already at Beginning-Of-Tape, a Check Status will be returned, and sense

information will have the EOM bit set.

Load Tape

---------

Assembler:	tapebios(0xACE, Load, 0L, (long)(retension))

'C'	 :	Tload((long)(retension));

The Load bit, if one, indicates the load function. The TAPE CONTROLLER

checks that the drive has been OFF Line and is now On Line and the tape is

positioned at the BOT marker. If this is the case the TAPE CONTROLLER assumes

that the operator has placed the correct tape in the drive, and returns a

GOOD STATUS. If the Drive has not been through the OFF Line - ON Line cycle

since the last tape movementm, or the tape is not at BOT, then the tape is

unloaded to force operator intervention, and a CHECK CONDITION status is

returned with the Sense Key set to Not Ready. Note that the TAPE CONTROLLER

requires the operator to perform the actual tape load sequence.

Unload Tape

-----------

Assembler:	tapebios(0xACE, Unload, 0L, (long)(retension))

'C'	 :	Tunload((long)(retension));

The Load bit, if zero, indicates the unload function, and requests

that the tape be unloaded from the drive. The drive is set OFF Line to allow

an operator to remove the tape. The Not Ready Sense Key is set.

Write File Mark

---------------

Assembler:	tapebios(0xACE, WrtFM, 0L, (long)(marks))

'C'	 :	Tfmark((long)(marks));

The Write Filemarks command causes the specified Number of Filemarks

to be written beginning at the current tape position. A zero in this field

indicates that no filemarks are to be written.

This command is also used to force any buffered data (see Buffered Mode

in the Mode Sense command) to be written. This command will not return GOOD

STATUS unless all buffered data blocks and the filemarks (if any) are correctly

written on the medium.

If the End of Tape marker is encountered while writing, an attempt to

finish writing any buffered data is made. The command will terminate with CHECK

CONDITION status and with the EOM bit set in the Extended Sense set to one. If

any filemarks remain to be written, then the Sense Key will be set to VOLUME

OVERFLOW. If the TAPE CONTROLLER is not buffered (Buffered Mode of the Mode

Sense command is zero), then the valid bit in Extended Sense will be set to one

and the Information Bytes will be set to the number of unwritten filemarks. If

the TAPE CONTROLLER is buffered (Buffered Mode of the Mode Sense command is

one), then the valid bit will be set to one and the Information Bytes will be

set to the total number of blocks not written i.e. the number of unwritten

filemarks plus the number of blocks remaining in the TAPE CONTROLLER's buffer.

Note that in this case, it is possible for the value in the Information Bytes

to exceed the Transfer Length.

If the Str bit in byte 5 of the command is set to one, the TAPE

CONTROLLER will return a GOOD STATUS byte immediately after issuing the last

write filemark command to the tape. This can be used to help maintain tape

streaming over multiple files.

Request Sense

-------------

Assembler:	tapebios(0xACE, Reqsen, *(long)(buffer),

(long)(bytecount))

'C'	 :	Trsense(*(long)(buffer), (long)(bytecount));

*

*	bios(0xACE, RSENSE, *(long)(buffer), (long)(bytecount));

*		Supply buffer address pointer

*		and bytecount of sense data (11)

The Request Sense command requests that the TAPE CONTROLLER transfer

Sense Data to the initiator.

The Sense Data will be valid for a CHECK CONDITION status returned for

the last command issued by the initiator. This Sense Data will be preserved

by the TAPE CONTROLLER for the initiator until retrieved by the Request Sense

command or until receipt of any other command from the initiator for the same

Logical Unit.

The Allocation Length specifies the number of bytes that the initiator

has allocated for the returned Sense Data. An Allocation Length of zero

Indicates that four bytes of Sense Data will be transferred. Any other value

indicates the maximum number of bytes that will be transferred. The TAPE

CONTROLLER will terminate the DATA IN phase when allocation length bytes have

been transferred or when all available Sense Data has been transferred to the

initiator.

The CHECK CONDITION status is only used to report fatal errors for this

command. For example:-

- The Target receives a non-zero RESERVED bit in the Command

Descriptor Block.

- An unrecovered parity error occurs on the DATA BUS.

- A SCSI Bus or TAPE CONTROLLER malfunction prevents return of the

Sense Data.

Following a fatal error on a Request Sense command, Sense Data may be

invalid.

Non-extended Sense

------------------

The format of the Sense Data is determined by the Error Class. Error

Classes zero to six use the Nonextended Sense Data format, Error Class seven

is described in the Extended Sense section.

All the Nonextended Sense Data format classes provide vendor unique

information and, as such, have limited value due to potential incompatibility

between SCSI devices. Since the Extended Sense is properly defined, and is

more suitable for sequential devices, It is used exclusively by the TAPE

CONTROLLER.

Extended Sense

--------------

Extended Sense Data Format

--------------------------

================================================================

BIT	|   7  |   6  |	 5  |	4  |   3	|   2  |   1	|	 0  |

BYTE	|      |      |	    |		 |	  |	 |	|	 |

================================================================

0	|Valid |   1	 1	1  |   0      0      0	    0  |

-------|-------------------------------------------------------|

1	|		 Segment Number 			    |

-------|-------------------------------------------------------|

2	|File- |      |	    |		 |				    |

|mark  | EOM  | ILI  |	0  |				     |

-------|-------------------------------------------------------|

3	|			Information Byte (MSB)	       |

-------|-------------------------------------------------------|

4	|			Information Byte			 |

-------|-------------------------------------------------------|

5	|			Information Byte			 |

-------|-------------------------------------------------------|

6	|			Information Byte (LSB)	       |

-------|-------------------------------------------------------|

7	|			Additional Sense Length	       |

-------|-------------------------------------------------------|

8	|			Additional Sense Bytes	       |

================================================================

Error Class seven specifies Extended Sense. Error Code zero specifies

the Extended Sense Data format.

The valid bit indicates that the information bytes contain valid

information as follows:

The information bytes normally specify the difference (residue)

between the requested length and the actual length in either bytes or blocks,

as determined by the command. (Negative values are indicated by two's

complement notation.)

The information bytes are not defined if the valid bit is zero.

The Segment Number is always set to 0.

The Filemark bit indicates that the command moved the tape over a

filemark.

The End of Medium (EOM) bit Indicates that an end of medium has been

detected by the tape drive. This bit indicates end-of-tape if the direction

was forward or beginning-of-tape if the direction was reverse.

The Incorrect Length Indicator (ILI) bit indicates that the requested

logical block length did not match the logical block length of the data on

the medium.

The Sense Keys are described in the tables that follow.

The Additional Sense Length specifies the number of Additional Sense

Bytes to follow. If the Allocation Length of the Command Descriptor Block is

too small to transfer all of the Additional Sense Bytes, the Additional Sense

Length is not adjusted to reflect the truncation.

The TAPE CONTROLLER sets one additional (Vendor Unique) byte to

provide information about the current status of the tape. This information

is generated at the time of the first Request Sense command. Subsequent

REQUEST SENSE command will not update this information.

Additional Sense Bytes

----------------------

=================================================================

BIT	|   7* |   6  |	 5  |	 4  |	3  |   2  |   1  |	 0	|

BYTE	|      |	    |	   |		 |	   |	  |	 |	|

=================================================================

8	| PROT |  EOT |  BOT |	   Soft Error Count		|

=================================================================

Sense Key (0-7) Descriptions

----------------------------

=================================================================

Sense Key     Description

---------     ---------------------------------------------------

0H		  NO SENSE. Indicates that there is no specific Sense

Key information to be reported. This would be the

case for a successful command or a command checked

because of Filemark, EOM or ILI bits being set to

one.

1H		  RECOVERED ERROR.	Indicates that the last command

completed successfully with some recovery action

performed by the target.

2H		  NOT READY. Indicates that the Tape Drive addressed

cannot be accessed. Operator intervention may be

required to correct this condition.

3H		  MEDIUM ERROR. Indicates that the target detected a

non-recovered error condition which was probably

caused by a flaw in the medium or an error in the

recorded data.

4H		  HARDWARE ERROR. Indicates that the target detected

a non-recoverable hardware failure (for example,

controller failure, device failure, parity error,

etc.) while performing the commandor during a self

test.

5H		  ILLEGAL REQUEST. Indicates that there was an Illegal

parameter in the Command Descriptor Block or in the

additional parameters supplied as data for some

commands (Set Mode, etc.).

6H		  UNIT ATTENTION. Indicates that the Tape Drive may have

been taken off line or the TAPE CONTROLLER reset (by

BUS DEVICE RESET message or "Hard" Reset Condition or

by CLEAR DRIVE command) since the last command was

issued to the Logical Unit. This Sense Key is reported

the first time that any command is issued after the

condition is detected and the requested command is not

performed. The UNIT ATTENTION Sense Key is reported to

all Initiators which subsequently issue a command to

the TAPE CONTROLLER. This Sense Key is cleared for the

next command from the same initiator.

7H		  DATA PROTECT. Indicates that a command which writes to

the tape was prevented by the absense of a write enable

ring. The write operation is not performed.

8H		  BLANK CHECK. Indicates that a read operation timed out

without finding data.

9H		  VENDOR UNIQUE. Provided by SCSI to report vendor unique

conditions.

AH		  COPY ABORTED. Generally not used.

BH		  ABORTED COMMAND. Indicates that the target aborted the

command. The initiator may be able to recover by trying

the command again.

CH		  EQUAL. not used.

DH		  VOLUME OVERFLOW. Indicates that the tape drive has

reached the End of Tape and data remains in the buffer

which has not been written to the tape. A Recover

Buffered Data command may be issued to read the

unwritten data from the buffer. ( set only when PEOT

is set in MODE SELECT command)

EH		  MISCOMPARE. not used.

FH		  This Sense Key is RESERVED.

Mode Select

-----------

Assembler:	tapebios(0xACE, Modsel, *(long)(parameter_list),

(long)(list_len))

'C'	 :	Tmselect(*(long)(parameter_list), (long)(list_len));

The Mode Select command provides a means for the initiator to specify

device parameters to the TAPE CONTROLLER.

The Parameter List Length specifies the length in bytes of the

Parameter List which will be transferred during the DATA OUT phase. The Length

of Parameter List, if zero, indicates no data will be transferred. This is

not considered an error condition.

The Parameter List contains a four byte header, followed by zero or one

eight byte Block Descriptors, followed by the VENDOR UNIQUE parameters.

The Buffered Mode is defined as follows:

0H	The TAPE CONTROLLER will not report GOOD STATUS on Write

commands until all data blocks are actually written

correctly onto the tape.

1H	The TAPE CONTROLLER will report GOOD STATUS on Write commands

as soon as the last (or only) data block has been received

by the TAPE CONTROLLER, therefore data may be resident in the

buffer until more data is sent, or a Write Filemark command

is issued.

2H-7H	Reserved.

Mode Select Parameter List

--------------------------

================================================================

BIT	|   7  |   6  |	 5  |	4  |   3	|   2  |   1	|	 0  |

BYTE	|      |      |	    |		 |	  |	 |	|	 |

================================================================

0	|   0		0	 0	0	    0	   0	  0		0  |

-------|-------------------------------------------------------|

1	|   0		0	 0	0	    0	   0	  0		0  |

-------|-------------------------------------------------------|

2	|RE-   | 		   |				 |

|SERVED|    Buffered Mode   |		Speed 	       |

-------|-------------------------------------------------------|

3	|			Block Descriptor Length	       |

================================================================

Block Descriptor

================================================================

0	|			Density Code			     |

-------|-------------------------------------------------------|

1	|			Number of Blocks (MSB)	       |

-------|-------------------------------------------------------|

2	|			Number of Blocks			 |

-------|-------------------------------------------------------|

3	|			Number of Blocks (LSB)	       |

-------|-------------------------------------------------------|

4	|   0		0	 0	0	    0	   0	  0		0  |

-------|-------------------------------------------------------|

5	|			Block Length (MSB) 		   |

-------|-------------------------------------------------------|

6	|			Block Length			     |

-------|-------------------------------------------------------|

7	|			Block Length (LSB) 		   |

================================================================

Two Vendor Unique parameter bytes follow the Block Descriptor

================================================================

0	|	     Vendor Unique  Byte 0			     |

-------|-------------------------------------------------------|

1	|	     Vendor Unique  Byte 1			     |

================================================================

The Speed is defined as follows:

0H	Default Speed (high speed)

1H	low speed

2H-FH	high speed

The Block Descriptor Length must be set to either 0 or 8 to specify

0 or 1 Block Descriptors. It is equal to the number of Block Descriptors times

eight. A Block Descriptor Length of zero indicates no Block Descriptors are

included in the Parameter List. This shall not be considered an error

condition.

The Block Descriptor specifies the medium characteristics for all or

part of a Logical Unit. Each Block Descriptors contains a Number of Blocks and

a Block Length.

The Density Code parameters are drive dependant. Consult the manual for

your tape drive to obtain the definitions for this field.

The Number of Blocks field may or may not be ignored by your TAPE

CONTROLER. Again, consult the manual for your tape drive.

The Block Length specifies the length in bytes of the logical block.

The Block Length, if zero, indicates the length is variable.

The next two bytes are Vendor Unique. Check the definition of these

bytes in the manual for your tape drive.

Mode Sense

----------

Assembler:	tapebios(0xACE, Modsen, *(long)(parameter_list),

(long)(list_len))

'C'	 :	Tmsense(*(long)(parameter_list), (long)(list_len));

The Mode Sense command provides a means for the TAPE CONTROLLER to

report its current operational mode. It is a complementary command to the Mode

Select command.

The Allocation Length specifies the number of bytes that the initiator

has allocated for returned Mode Sense Data. An Allocation Length of zero

indicates that no Mode Sense Data will be transferred. This is not considered

an error condition. Any other value indicates that number of bytes or less

will be transferred. The TAPE CONTROLLER will terminate the DATA IN phase when

Allocation Length bytes have been transferred or when all the Mode Sense Data

has been transferred to the initiator.

NOTE: Check the manual for your tape drive. Some drives report back to the

initiator with CHECK CONDITION if specific values for the Allocation Length

are not specified.

The Mode Sense Data contains a four byte header, followed by one eight

byte Block Descriptor followed by the VENDOR UNIQUE parameters.

The Sense Data Length specifies the length in bytes of the Mode Sense

Data which is available to be transferred during the DATA IN phase.

The Medium Type is always set to zero indicating only one medium type

is supported.

The Write Protected (WP) bit, if zero, indicates the tape is write

enabled. The Write Protected bit, if one, indicates the tape is write

protected.

Mode Sense Data

---------------

================================================================

BIT	|   7  |   6  |	 5  |	4  |   3	|   2  |   1	|	 0  |

BYTE	|      |      |	    |		 |	  |	 |	|	 |

================================================================

0	|		Sense Data Length			     |

-------|-------------------------------------------------------|

1	|			Medium Type			    |

-------|-------------------------------------------------------|

2	|  WP  |   Buffered Mode    |		 Speed	       |

-------|-------------------------------------------------------|

3	|			Block Descriptor Length	       |

================================================================

Block Descriptor

================================================================

0	|			Density Code			     |

-------|-------------------------------------------------------|

1	|			Number of Blocks (MSB)	       |

-------|-------------------------------------------------------|

2	|			Number of Blocks			 |

-------|-------------------------------------------------------|

3	|			Number of Blocks (LSB)	       |

-------|-------------------------------------------------------|

4	|   0		0	 0	0	    0	   0	  0		0  |

-------|-------------------------------------------------------|

5	|			Block Length (MSB) 		   |

-------|-------------------------------------------------------|

6	|			Block Length			     |

-------|-------------------------------------------------------|

7	|			Block Length (LSB) 		   |

================================================================

Two Vendor Unique parameter bytes follow the Block Descriptor

================================================================

0	|		Vendor Unique  Byte 0			 |

-------|-------------------------------------------------------|

1	|		Vendor Unique  Byte 1			 |

================================================================

The Buffered Mode and Speed field are defined in the Mode Select

Command.

The Block Descriptor Length bytes is set to 08H to indicate 1 Block

Descriptor (of 8 bytes) is available.

The Density Code is defined in the Mode Select Command.

The Number of Blocks field is always set to zero, indicating all the

remaining logical blocks of the Logical Unit

The Block Length specifies the length in bytes of the Tape block. This

is initialised at power up to be zero, indicating the length is variable, but

can be updated by the Mode Select Command to provide a fixed block size. This

value is maintained solely by the Mode Select Command, and is not affected by

the size of any data block read or written to tape.

The following two bytes are Vendor Unique.

Space

-----

Assembler:	tapebios(0xACE, Space, 0L, (long)(code/count))

'C'	 :	Tspace((long)(code/count));

The Space command provides a variety of tape positioning functions

which are determined by the Code and Count. Both forward (toward EOT) and

reverse (toward BOT) positioning are provided.

The Code is defined as follows:

DB(1)	DB(0)	Description

0	0	Blocks

0	1	Filemarks

1	0	Sequential Filemarks

1	1	Physical End-of-Data

When spacing over Blocks or Filemarks the Count field specifies the

number of blocks or filemarks to be spaced over. A positive value N in the

Count field causes forward tape movement over N blocks or filemarks ending on

the EOT side of the last block or filemark. A zero vlaue in the Count field

causes no tape movement. A negative value -N (2's complement notation) in the

Count field causes reverse tape movement over N blocks or filemarks ending

on the BOT side of the last block or filemark.

NOTE: Not all streamers support the BACKSPACE function, experiment !!!!!

If a filemark is encountered while spacing over Blocks, tape movement

is stopped. The tape will be positioned on the EOT side of the filemark if

movement was in the forward direction and on the BOT side of the filemark if

movement was in the reverse direction. CHECK CONDITION status will be sent

to the initiator and the Filemark and Valid bits in Extended Sense will be

set to one. The Information Bytes will be set to the difference (residue) in

the requested Count and the actual number of blocks spaced over (not including

the filemark).

If the EOT marker is encountered while spacing forward over Blocks or

Filemarks, the TAPE CONTROLLER will return CHECK CONDITION status to the

initiator and will set the End-of-Medium (EOM) bit in Extended Sense to one.

The Sense Key will be set to MEDIUM ERROR. The TAPE CONTROLLER will set the

valid bit to one and the Information Bytes to the difference (residue) between

the requested Count and the actual number of blocks or filemarks spaced over.

If the BOT marker is encountered while spacing over Blocks or Filemarks

in the reverse direction, the TAPE CONTROLLER returns CHECK CONDITION status to

the initiator and sets the End-of-Medium (EOM) bit in Extended Sense to one.

The Sense Key will be set to NO SENSE. The TAPE CONTROLLER will set the valid

bit to one and the Information Bytes to the difference (residue) between the

requested Count and the actual number of blocks or filemarks spaced over.

When spacing over Sequential Filemarks, the Count field is interpreted

as follows:

- A positive value N causes forward tape movement to the first

occurence of N or more consecutive filemarks stopping after the Nth

filemark.

- A zero value shall cause no tape movement.

- A negative value -N (2's complement notation) causes reverse tape

movement to the first occurence of N or more consecutive filemarks

stopping on the BOT side of the Nth filemark.

When spacing to physical End-of-Data, the Count field is ignored.

Forward tape movement occurs until the TAPE CONTROLLER has passed over blank

tape for approximately 10 seconds. The tape is then repositioned to the point

immediately after the last data block or filemark read. This leaves the tape

positioned so that a subsequent write command would append data to the last

recorded information on the tape.

Erase

-----

Assembler:	tapebios(0xACE, Erase, 0L, 0L)

'C'	 :	Tspace();

The Erase command causes part or all of the remaining tape to be erased

beginning from the current tape position.

The distance to be erased is controlled by the Long bit. The Long bit,

if one, indicates all remaining tape is erased. If zero, it indicates that a

standard length of tape is erased. The length in this case is governed by the

tape formatter, but is typically 4 inches of tape.

Some other tape controllers (especially those concerned with 1/4 inch

cartridges) may only be able to erase a tape from BOT (i.e. before writing to

the tape). Programmers should therefore only issue an erase command from BOT.

Inquiry

-------

Assembler:	tapebios(0xACE, Inquire, *(long)(buffer),

(long)(bytecount))

'C'	 :	Tinquir(*(long)(buffer), (long)(bytecount));

The Inquiry command requests that information regarding parameters of

the TAPE CONTROLLER be sent to the initiator.

The Allocation Length specifies the number of bytes that the initiator

has allocated for returned Inquiry Data. An allocation Length of zero indicates

no inquiry data shall be transferred. This is not considered an error

condition. Any other value indicates the maximum number of bytes to be

transferred. The TAPE CONTROLLER terminates the DATA IN phase when this number

of bytes have been transferred or when all available Inquiry Data has been

transferred to the initiator.

The CHECK CONDITION status will only be reported when the TAPE

CONTROLLER cannot return the Inquiry Data.

The Inquiry Data contains a five byte header, followed by the VENDOR

UNIQUE parameters, if any.

The Peripheral Device Type is set to 01 to indicate a sequential

device, unless the Logical Unit Number set in the command is not valid (i.e.

not 000) In which case this byte is set to 7FH to indicate the Logical Unit

is not present.

The Removable Medium (RMB) bit is set to 1 to indicate the medium is

removable.

Inquiry Data

------------

================================================================

BIT	|   7  |   6  |	 5  |	4  |   3	|   2  |   1	|	 0  |

BYTE	|      |      |	    |		 |	  |	 |	|	 |

================================================================

0	|			Peripheral Device Type	       |

---------------------------------------------------------------|

1	| RMB  | 	     Device Type Qualifier		    |

---------------------------------------------------------------|

2	|			Version				 |

---------------------------------------------------------------|

3	|   0		0	 0	0	    0	   0	  0		0  |

---------------------------------------------------------------|

4	|			Additional Length			  |

===============================================================|

5	|	    Custom Version number (see text)	       |

---------------------------------------------------------------|

6	|   0		0	 0	0	    0	   0	  0		0  |

---------------------------------------------------------------|

7	|   0		0	 0	0	    0	   0	  0		0  |

---------------------------------------------------------------|

8 - 35 | (TAPE CONTROLLER's title and revision details, ASCII) |

---------------------------------------------------------------|

36 ->	| (Vendor Unique parameters (see text)		       |

================================================================

The Device Type Qualifier is a seven bit user specified code. This code

is normally set to 0.

The Version if the implemented version of the SCSI standard and is

defined in the TAPE CONTROLLER as follows:

00H	Pre-release firmware

01H	Standard release firmware. This code is used by SCSI Devices

who claim to comply with SCSI rev 17B.

The Additional Length specifies the length in bytes of the extra and

VENDOR UNIQUE parameters.

Byte 5 will normally be set to 00H unless customized firmware is

supplied. Bytes 8 to 35 generate the TAPE CONTROLLER's title and revision

details in accordance with the SCSI Command Command Set Specification.

The remaining bytes are Vendor Unique.

Alan Hourihane.

email: alanh@logitek.co.uk

17/3/92.