******************************************************************

*                DMA Sound Oscilloscope V2.20                    *

*                 (C) 1994 Terrosaur Systems                     *

******************************************************************

* This program is FreeWare. It may be distributed freely for non *

* profit purposes only. The author retains the copyright.        *

******************************************************************

--------------------

System requirements:

--------------------

Falcon030, any TOS, any screen res except true colour.

(optional) Background tracker MOD player.

----------------

What Does it Do?

----------------

DMA_OSC displays two oscilloscope traces (stereo left and stereo

right) or a Lissajon figure of the combined traces in real time.

DMA_OSC runs in one of two modes:

1) (default): MOD mode. In this mode, the traces will correspond

to any soundtracker type MOD currently being played in the

background by a seperate player.

2) MIC mode. In this mode, the traces will correspond to sound

being input through the microphone socket on the rear of the

Falcon.

------------------

Using the program.

------------------

When first run, the program will display a window with two traces

and a menu bar. The colour palette is changed to provide a clearer

display of the traces.

What is displayed by the traces depends on whether a MOD player is

running in the background. If one is, the traces will correspond

to the music being played. If not, they will display nothing, or

random garbage.

To use the program in MOD mode, you should make sure a background

MOD player is loaded before you run DMA_OSC. A desk accessory MOD

player is best as this can then be accessed from the

'Oscilloscope' menu in DMA_OSC. I recommend Paula 2.2a as this

produces by far the best quality output.

Once the MOD player is installed, you start it and run

DMA_OSC2.PRG. You will not have to adjust anything else.

To use the program in MIC mode, first connect your sound source to

the microphone socket on the rear of the machine. Then run

DMA_OSC2.PRG. Select the MIC mode option from the Options menu.

The traces will now correspond to the sound being played into the

microphone socket.

You can change between MOD and MIC modes at any time using the

Options menu.

To change from trace mode to Lissajon mode, click the 'fuller'

button at the top right of the window. Lissajous mode works best

with MIC mode.

To exit, select Quit from the Options menu or click the window

close button.

--------------

Technical bits

--------------

For those who are interested, this is how the program works.

In MOD mode, the program locates the DMA buffer currently being

used and uses the volume at each point in the sample to determine

the coordinates of the corresponding point on the screen. To

create the traces, the stereo left channel is used to produce the

top trace, and stereo right to produce the bottom trace. The

volume is proportional to the displacement on the Y-axis. To

create the Lissajon figure, the volume of the left channel is used

to calculate the Y displacement, and the volume of the right

channel is used to calculate the X displacement.

In MIC mode, much the same calculations occur, except that the

sound information is obtained by sampling blocks into a buffer

from the microphone socket.

-----

Notes

-----

Speed:

------

In order to achieve a useable speed, the program has to write

directly to the logical screen RAM rather than using the VDI. This

has two drawbacks:

1) The display will freeze if another window is opened on top, eg

by a desk accessory. This is because without using the VDI it is

difficult to clip the display to those parts of the program window

which are visible. I have tried several other clipping techniques,

none were fast enough. Therefore, DMA_OSC will only update the

display if its window is the top window.

2) The window cannot be sized or moved. The program works out the

biggest window it can have when first run and uses that.

The display will always update in real time. The more processor

time the program gets, the more frames per second it will draw and

vice-versa. So if your MOD player uses a lot of processor time,

the display will appear sluggish. The program was designed with

Paula 2.2a in mind as a MOD player, and Paula will play at 50kHz

or 25KHz interpolated (better than 50KHz quality) using about 50%

of processor time. This gives an adequate display updating 15-20

times per second. In MIC mode of course, it has all the processor

time and gives 40-50 frames per second.

MultiTOS:

---------

DMA_OSC2.PRG is fully GEM legal apart from the display, and will

run under MultiTOS. The lack of a sizable, moveable window makes

it not very practical to do so, though, and also the speed of the

display will suffer under MultiTOS.

True Colour:

------------

The display routines in DMA_OSC have been heavily optimised for

high speed on a bit-plane display. As true colour modes use a

different screen RAM format, DMA_OSC2 will not run in true colour.

MOD players:

------------

DMA_OSC2.PRG is designed with desk accessory mod players in mind,

such as Paula 2.2a, DeskTrackerMT, Ultimate tracker etc, which can

be accessed from the GEM menu bar. These can be started before or

whilst DMA_OSC is running. It will work with any background

player, but obviously if it isn't an ACC you will not be able to

access it whilst DMA_OSC2 is running.

Display:

--------

DMA_OSC2 will open the largest window it can and read sufficent

data each frame to fill the window horizontally. This means that

run in a res of 640x480, it will read 1280 bytes each frame, to

create the two 640 pixel traces. In some cases, the buffer used

for the sample by the MOD player may be smaller than this. For

example, Paula 2.2a in 25kHz mode uses a 1024 byte buffer. This

means that the last part of the trace may be blank or garbage.

There is no easy fix for this, especially as some players will

dynamically alter their buffer to fit the current MOD file.

At present DMA_OSC2 only supports 8 bit stereo replay. 16 bit

players will cause corrupted traces.

MIC control:

------------

The program has no direct control for sound volume control, or

gain control for the microphone socket. However, this can be

easily adjusted by using the SOUND.CPX with XCONTROL.ACC.

Possible future developments:

-----------------------------

As it stands, the program does pretty much what I wanted it to do,

and works perfectly on my system. However if I get a sufficient

response to this release, I may add the following features in a

future release:

Moveable/sizeable window

Alternative colour schemes

A run-other-program option for non-acc mod players

Support for 16 bit mod players

True colour support

Feedback:

---------

Please send feedback on your views on this program to:

Internet: cs93dlh@dcs.exeter.ac.uk

D.L.Hastings@exeter.ac.uk

FidoNet:  2:440/108.6

TurboNet: 100:1011/0.6

NeST:     90:102/140.6

AtariNet: 51:502/100.6

Snail mail: D. Hastings, Aspenglow, Rectory Road, Alderbury,

Salisbury, Wilts. SP5 3AD (England)

Disclaimer:

-----------

The author takes no responsibility for any damage caused by the

use, non-use, or mis-use of this program.