The IBM PC 6800/01/04/05/09/11 cross assemblers

(Now also for Atari ST)

GENERAL

The assemblers are named as*.exe (as*.ttp for Atari ST) where '*'

is  any of 0,1,h1,4,5,9 or 11 depending on which one you're using.

Command line arguments specify the filenames to assemble.

The assemblers accept options from the command line to be

included in the assembly.  These options are the following:

l	   enable output listing.

nol    disable output listing (default).

cre    generate cross reference table.

s	   generate a symbol table.

c	   enable cycle count.

noc    disable cycle count.

The command line looks like this :

as* file1 file2 ... [ - option1 option2 ...]

If this method of passing commands to the assembler is

used rather than the OPT pseudo op code, a space should

separate the minus sign from the last file name and the

first option.  Example:

as5 program -l cre

This command assembles file 'program' with an output

listing and a cross reference table.

The `S1' formatted object file is placed in file

`filename.S19', the listing and error messages are

written to  the standard output.  If multiple files are

assembled, the 'S1' file will be placed under the first

file's name.S19.

The listing file contains the address and bytes assembled for each

line of input followed by the original input line (unchanged, but

moved over to the right some).	If an input line causes more than 6

bytes to be output (e.g. a long FCC directive), additional bytes

(up to 64) are listed on succeding lines with no address preceding

them.

Equates cause the value of the expression to replace the address

field in the listing.

Equates that have forward references cause Phasing Errors in Pass

2.

Expressions may consist of symbols, constants or the character	'*'

(denoting the current value of the program counter) joined together

by one of the operators: +-*/%&|^.  The operators are the same as

in C:

+	add

-	subtract

*	multiply

/	divide

%	remainder after division

&	bitwise and

|	bitwise or

^	bitwise exclusive-or

Expressions are evaluated left to right and there is no provision

for parenthesized expressions.	Arithmetic is carried out in signed

twos-complement integer precision (16 bits on the IBM PC, also 16

bit on the Atari ST)

Constants are constructed with the same syntax as the Motorola MDOS

assembler:

'       followed by ASCII character

$	followed by hexadecimal constant

@	followed by octal constant

%	followed by binary constant

digit	decimal constant

ERRORS

Error diagnostics are placed in the listing file just before the

line containing the error.  Format of the error line is:

Line_number: Description of error

or

Line_number: Warning --- Description of error

Errors of the first type in pass one  cause  cancellation  of  pass

two.  Warnings	do  not  cause	cancellation of pass two but should

cause you to wonder where they came from.

Error messages are meant to be self-explanatory.

If more than one file is being assembled, the  file  name  precedes

the error:

File_name,Line_number: Description of error

Finally, some errors are classed as fatal and  cause  an  immediate

termination  of  the assembly.	Generally these errors occur when a

temporary file cannot be created or is lost  during  the  assembly.

Consult your local guru if this happens.

DIFFERENCES

For indexed addressing, the comma is required before the  register;

`inc x' and `inc ,x' are not the same.

Macros are not supported.  (try M4 or M6)

The force size operators ('>' and  '<')  are  implemented  for  all

assemblers.

The only pseudo-ops supported are:

ORG, FCC, FDB, FCB, EQU, RMB, BSZ, ZMB, FILL

PAGE and OPT.

The OPT pseudo-op allows the following operands:

nol	Turn off output listing

l	Turn on output listing (default)

noc	Disable cycle counts in listing (default)

c	Enable cycle counts in listing (clear total cycles)

contc	Re-enable cycle counts (don't clear total cycles)

cre	Enable printing of a cross reference table

s	generate a symbol table

Some of the more common pseudo-ops are not present:

SPC	Use blank lines instead

END	The assembly ends when there is no more input

TTL	use `pr' to get headings and page numbers

NAM[E]	Did you ever use this one anyway?

The above 4 pseudo-ops are recognized, but ignored.

ZMB (Zero Memory Bytes) is equivalent to BSZ (Block Store  Zeroes).

FILL can be used to initialize memory to something other than zero:

FILL val,nbytes.

TARGET MACHINE SPECIFICS

(as0)	6800:	Use for 6802 and 6808 too.

(as1)	6801:	You could use this one for  the  6800  and  avoid

LSRD, ASLD, PULX, ABX, PSHX, MUL, SUBD, ADDD, LDD

and STD.

(as4)	6804:	The symbols 'a', 'x' and 'y'  are  predefined  as

$FF, $80 and $81 respectively.	Also  defined  as

'A',   'X'   and   'Y'.    Because  of  the  6804

architecture, this means that 'clr x'  will  work

since  the  x register is just a memory location.

To  use  short-direct  addressing,   the   symbol

involved  must	not  be a forward reference (i.e.

undefined) and must be in the range $80-$83.

Remember  that	bytes  assembled  in  the   range

$10-$7F  will go into the data space; There is no

program space ROM for these locations.

The syntax for Register indirect addressing is as

follows:

menmonic [<x>or<y>]

an example is:

lda [x]

the comma ',' is not allowed.

The MVI instruction (move immediate) has its own

format :

mvi address,#data   where address is

an 8-bit address in page zero, and data is

the value to be written to specified location.

(as5)	6805:	There is no 'opt cmos' pseudo, so be careful  not

to use STOP or WAIT in a program that is destined

for  an  NMOS  version	of  the  6805.	 The  MUL

instruction   should   also  be  avoided  on  all

versions of the 6805 except the C4.  Cycle  times

are for the NMOS versions.

(as9)	6809:	The SETDP pseudo-op is not implemented.

Use the '>' and '<` operators to force the size of

operands.

For compatibility, CPX is equal to CMPX.

(as11) 68HC11: Bit manipulation operands are separated by  blanks  instead

of commas since the 'HC11 has bit manipulation instructions

that operate on indexed addresses.

DETAILS

Symbol:  A string of  characters  with	a  non-initial	digit.	The

string of characters may be from the set:

[a-z][A-Z]_.[0-9]$

( . and _ count as non-digits ).  The `$' counts as a digit

to   avoid   confusion	with  hexadecimal  constants.	All

characters of a symbol	are  significant,  with  upper	and

lower  case  characters being distinct.  The maximum number

of characters in a symbol is currently set at 15.

The symbol table has room for  at  least  2000	symbols  of

length 8 characters or less.

Label:	A symbol starting in the first column is a  label  and	may

optionally  be	ended  with a ':'.  A label may appear on a

line by itself and is then interpreted as:

Label	EQU	*

Mnemonic:  A symbol preceded by at least one whitespace  character.

Upper  case characters in this field are converted to lower

case before being checked as a legal mnemonic.	Thus `nop',

`NOP' and even `NoP' are recognized as the same mnemonic.

Note that register names that sometimes appear at  the	end

of  a  mnemonic (e.g. nega or stu) must not be separated by

any  whitespace  characters.   Thus  `clra'   means   clear

accumulator A, but that `clr a' means clear memory location

`a'.

Operand:  Follows mnemonic, separated by at  least  one  whitespace

character.   The   contents   of   the	 operand  field  is

interpreted by each instruction.

Whitespace: A blank or a tab

Comment:  Any text after all operands for  a  given  mnemonic  have

been  processed or, a line beginning with '*' up to the end

of line or, an empty line.

Continuations:	If a line ends with a backslash (\) then  the  next

line  is  fetched  and	added to the end of the first line.

This continues until a line is seen which doesn't end in  \

or  until  MAXBUF characters have been collected (MAXBUF >=

256 ).

FILES

filename.S19	S-record output file

STDOUT		listing and errors (use redirection for listing file)

Fwd_refs	Temporary file for forward references.

For the Atari ST, it is better to use these assemblers from within

a CLI. This way you can redirect the STDOUT to a file if you require

a listing file.

IMPLEMENTATION NOTES

This is a classic 2-pass assembler.  Pass 1 establishes the  symbol

table and pass 2 generates the code.

12/11/84 E.J.Rupp

This version of the cross assemblers ported to the IBM PC 4/13/87

This latest version ported to Atari ST 16/12/90 By N.Benci.

Atari ST notes

All of the assemblers were tested using a mixture of mnemonics.

As far as possible they seemed to produce the correct code. I have

only tested the 6809 code. This was OK. The port was direct, ie;

now additions or deletion to the original code. The source code

is included for your perusal. If you improve on the code please

send either binaries or source to comp.*.atari.st so that others

may benefit.

The port was performed using HI-TECH C, a reasonable ANSI C

compiler. Although the code is not strictly ANSI C, not too many

complaints were forthcoming from the compiler.

Nino Benci. email - rdt154k%monu6.cc.monash.edu.au@uunet.uu.net