PAKHOST                      - page 1 -

+++++++++++++++++++++++++++++++++++

+                                 +

+     OPERATING INSTRUCTIONS      +

+                                 +

+                                 +

+             FOR                 +

+                                 +

+                                 +

+             PAKHOST             +

+                                 +

+                                 +

+++++++++++++++++++++++++++++++++++

2. 1.1994     by DF1GW

By no means should an operating guide be required for the pro-

gram.  Nevertheless  a  few tips for the operation are as fol-

lows:

Dear (X)YL's and OM's

Pakhost is free software only for radio amateurs. The  program

may  not be used commercially, even in part. All rights remain

with the Author.

PAKHOST IMPLEMENTS THE MODES  OF   PACKET,   BAUDOT,   ASCII,

AMTOR,  NAVTEX,  TDM,  CW AND  PACTOR.

The aim of the program is to operate the digital modes  simply

and  quickly.  The  change  between  the modes can be effected

simply and  quickly. The user should not  lose  sight  of  the

parameters.

PAKHOST is distributed by post direct from the author (format-

ted  disk  and  SASAE  please). The program may be distributed

between  licenced  amateurs  providing the files  PAKHOST.PRG,

PAKHOST.RSC,  PH_DOC.???  are  included.  Material and copying

costs may be recovered.  For  special  versions  or  listings,

please ask the author.

THE SALE OF PAKHOST IS NOT PERMITTED

PAKHOST is a terminal program for the Atari ST connected to an

AEA PAKRATT PK232. The pakratt will be activated in  hostmode.

PAKHOST is written in GFA-Basic 3.50-D.

PAKHOST will support all Pakratt firmware versions. The  mail-

box  commands  function for the Pakratt software versions from

31.10.89 and extended modes (navtex,TDM)  from  19.07.90  .The

Amtor mailbox from 01.08.91 is also included, as the mode pac-

tor from 05.03.93.

Please  send  Error  reports  and  suggestions   directly   to

me (DF1GW) in the mailbox DB0GV.

de df1gw

PAKHOST                      - page 2 -

Special thanks for the backing given by

AEA, Lynwood, Washington and

Stabo Elektronik GmbH.& Co. KG, Hildesheim.

Thanks also to.......

Volker DF3IAC, Weinheim who designed PAKHOST.RSC

Harald DL8LAP, Raisdorf who has given  constructive  criticism

and also converted at the same time  program sections ie PRG.-

START and ASCII.LISTEN.

DENIS  G0KIU, Nottingham who has translated the docs file into

english

for STEVE, G0FRV, Leicester to type up.

And  naturally thanks to also to the many OM's who with criti-

cism and contributions have made possible the progress of  the

program.

Have fun and success with PAKHOST !!

73'S DE WOLF, DF1GW

Wolfgang Mahlke; DF1GW

Im Steiles 10,

69469 Weinheim

BBS: DB0GV <--------------Please.

INSTALLATION AND START

Please do not use the original diskette as a working diskette.

Before  you  first  use  PAKHOST please format a few disks and

copy at  least  the  following.....PAKHOST.PRG;  PAKHOST.RSC;

DESKTOP.INF.  The  operating instructions (PAKHOST.DOC) can be

printed directly to most printers  directly  from  the   atari

desktop.

Your Atari should be set with the following parameter:

9600 Baud

8 Bits,1 Stopbit

Handshake (possible) out

With old Atari TOS versions  the   handshake  must

allways  be  switched OFF (with the supplied DESK-

TOP.INF). Do not activate any Soft/Firmware hands-

hake

If  you  use an Atari-STE or -TT you can connect your PK232 to

the ' MODEM2'-connector. In that case, you  must  change  the

parameter  '*04*MODEM1'  in the file FTAST.DAT to '*04*MODEM2'

with any text-editor.

The program PAKHOST.PRG can be started from the GEM-Desktop by

double click.

After  the  program  has  started the Pakratt will be switched

into "Hostmode".If the Pakratt was not in Host Mode and  still

has received data stored, this will be written into  the  file

de df1gw

PAKHOST                      - page 3 -

GULP.ASC.

If the Pakratt  clock was not set, it will now be set from the

Atari. If the Atari clock also was not set then you must enter

the correct time and date in the Pakratt  Field  appearing  in

the Dialogue box.

Subsequently  diverse  Installations  and  checks will be pre-

viously  prepared. Missing files will be (if also empty) crea-

ted by request. (ie  FTAST.DAT will be created with the  first

start).

The  CText can be adapted from the program, if you want diffe-

rent  responses  to your partner, depending of PAKHOST is run-

ning or not. Pakratt extracts the "active-text" from the  file

FTAST.DAT(*01*).  If nothing is entered there, nothing will be

adapted.

Finally the "Main Window" (welcome) appears.

When  you  start  PAKHOST  for the first time, please call the

pulldown help menu using the " HELP" key and click it in  the

position "Pakratt" in the field labelled "INIT".

With this a few fixed installations  will  be  undertaken  and

later  installations will only be required if you have changed

the parameters using another terminal program.

Closing  down with "QUIT" and clicking PAKHOST will return you

to the program help menu.

You should set your personal parameters ie "MYCALL" before the

first operation with the Pakratt.

And off we go ..............

MAIN WINDOW (WELCOME)

No version number is given for the  program  only  the  update

date which will be displayed in the main window.

From this window one can either jump direct to  the  displayed

mode's corresponding terminal (only press return) or change to

the mode required using the Function Keys F1-F5.

In  the first case it is able to go immediately and in the se-

cond case you have the option of changing the operating  para-

meters.

With two Cr's in the parameter window you will arrive at the

corresponding terminal.

In  the  Head line, which also remains for the duration of the

terminal Operation there will be  displayed  with  significant

short  form  whether  Paket-lite  is  switched  on;  the PK232

mailbox is active; the log loaded and eventually the automatic

log activated and whether the protocol has been switched on.

F1-F6:

switches  the PK232 to the desired mode and shows the main pa-

rameters.

Shift F1:

For the friends of GEM: Volker Hilenstein, DB4IN  has  adapted

the  parameter  installation  himself  and now a GEM Parameter

field for Packet-Radio has been integrated for our mouse fans.

F9

This activates the routine for the  acquisition  of  the   LOG

de df1gw

PAKHOST                      - page 4 -

BOOK. This can be read from any database-program.

L:

This by choice will switch to a slower output of received text

if you need to be able to read it.Do not use  this  unnecessa-

rily  because  the  program will react more slowly to the key-

board input than usual.

:

This will switch on and off the "umlaut" modifier but the ori-

ginal text will be saved.

r:

This  will switch on the Remote Control Facility which enables

the connected station to control the ATARI using certain  com-

mands. Please use only for tests.

x:

This will turn the TX On or Off.

HELP:

This will call the pulldown help menu's

UNDO:

This  will  exit  the program with a confidence request; ie Do

you really want to exit ?.

The Pakratt is then able to be left in  either  HOST  MODE  or

returned to the NORMAL MODE.

There  follows  a few alert boxes; ie is the monitor to be set

to 0?; Moncon to 1;and Whynot off ?.The  CText  will  also  be

changed  and the Passive Text in FTAST.DAT (*02*) will be loa-

ded.

If you so choose the  Mailbox can be activated by means of the

mouse click.

If  the  program should crash then the data recovery part will

not be executed. At best,start PAKHOST.PRG once again and exit

"decently" with UNDO. If  you  have  no  help  with  a  normal

terminal program then type the following:

Ctrl-a Ctrl-a O H O N CTRL-w

If  all  is  ok and you are in Command Mode then type in "Host

Mode Off".

PARAMETER WINDOW

Only the parameters for the respective mode will be displayed.

The parameters allow themselves to be altered by the typing of

the corresponding 2 part number. If possible the original com-

mands  will be shown. So you can use the Pakratt-manual to get

their meanings. With that  PAKHOST  reacts  possibly  intelli-

gently  about the respective demand. ie. Pakhost asks you whe-

ther you do not intend to alter PAKLITE and the baud rate,  if

you  are  in  packet  use  the command VHF to change the shift

width.

The construction of the window appears  somewhat  slow  for  a

computer  as fast as the Atari. This is because all of the pa-

rameters have to be read out of the Pakratt before  being  di-

splayed and after alteration all will be re-read. It is neces-

sary that for this reason, that  ie.  the  alteration  of  the

shift in Baudot, also affects the installation of the standard

conditions  of  the  Pakratt.  Test it out, whether it changes

only the shift in Baudot, does the Pakratt  invert  the  TX/RX

de df1gw

PAKHOST                      - page 5 -

signal.  This  inconvenience  therefore affects the mode inte-

grity.

In packet-radio,  amtor and pactor the mailbox parameters will

be  called  by  means  of the dots "29". A second packet radio

parameter window opens itself after "19".

RETURN:

With this the window is closed and the Pakratt will be left in

the corresponding mode.

de df1gw

PAKHOST                      - page 6 -

Terminal Window (all):

This is completely simple; Read above: Write below.

All  entered  control  characters  will  be  displayed. If the

Log Book; Pakhost Function; Protocol; Remote  Control;  or  if

the  Mailbox  is  activated; these will all be displaye in the

title line along with the following advice.

F1 to F8:

standard text

F9:

Insertion of partners callsign.Issue with shift F9

F10:

Toggle between send and recieve. The function of this key is

depends on the used mode.

Shift F9:

Issue for the partners callsign and when activated from out of

the main window with F9 and this is also for the aquisition of

the log book.

Shift F10:

This is used to set up the printer or disk save operation.

HELP:

This  shows the loaded standard texts on one great window. The

terminal window returns back again automatically if  an  input

follows.

Insert:

for fast switching to another mode. You will not see the  pa-

rameters.  This  point  is to react faster, when receiving on

SW. pse type 1 for packet-radio  (monitoring),  2  for  rtty-

baudot,  3 for rtty-ascii, 4 for amtor (listen), 5 for cw and

6 for pactor (listen).

Backspace:

This  clears the previous letter and sets the  cursor back, if

this is possible.

Clr/Home:

This  clears the pre-write bufferboth in the atari and pakratt

and the sending of open files will be stopped.

Undo:

This switches back to the main window.

Cursor-keys:

you  may scroll the rx-window one screen window, or to the be-

ginning or end of te buffer.

Sometimes  you  will get a scrambled rx-window, if the partner

station sends no or too much Cr's/Lf's. You can reorder it  by

pressing the rifht arrow.

Terminal-Window Packet:

title recieve window:

This gives an  overview of the 6 possible connections.

title send window:

This indicates the installed connection.

recieve window:

This  will  indicate  the connect reports, which another a the

active channel affects. NOTE ! this will not be saved.

send window:

The text to be sent out will be sent after pressing  "Return".

As  long  as  a line is not completed or "Return" has not been

de df1gw

PAKHOST                      - page 7 -

pressed it can be corrected using "backspace"

F10:

This key is used to Connect or Disconnect on  the  appropriate

channel. If instead the "partner" calls "Help" will be issued,

up to 10 pre-installed standard connects via the function keys

can  be  called.  These  Standard connects will be defined  in

"FTAST.DAT" (*70* - *79*). Also the same "Help" functions with

the Path Request (via ?). The Standard Paths will  be  defined

in "Ftast.Dat" (*80* - *89*).

shift F1-F6:

This will allow you to toggle between the 6 possible channels.

All the connected channels are shown on the Top  line  in  the

main window along with the station connected to that channel.

shift F7:

This will display the "MONITOR WINDOW"

shift F8:

This will only function on connected channels and the  respec-

tive channel will be fixed (ie CONperm ON).In the title window

will   be   displayed  "fixed  connected  with........".  This

function prevents the connection from being severed  by  your-

self  or  the  other station.By pressing "shift F8" again this

will be removed (ie CONperm OFF). NOTE! I have found  that  if

you  press  this whilst trying to connect to a station it WILL

NOT allow the connection to be broken and will keep trying  to

reconnect. (steve G0FRV)

Monitor Window

This window is called using "Shift F7" and when in this window

the following functions are available:

F1-F7:

these  will  direct the parameter installation as given in the

send window

F8:

This will show the status of the 6 packet radio channels.

F9:

This will request the MHeard list from the Pakratt.

F10:

This will show a connect list with 10 standard connects  which

are  available  from  the  function keys. These are created by

Pakhost from the monitor window. All you have to do is  choose

from the list and press the appropriate F-key.

Terminal Window Baudot or Ascii:

Title Receive Window:

The most important parameters for rtty are: -baud rate;  Shift

width; Shift polarity; for both send and receive.

Send Window:

The  outgoing text will be sent when either the "space bar" or

"return key" is pressed. As long as the word has not been com-

pleted it is able to be corrected.

CTRL-D:

This  will  be  inserted in the normal text and will switch to

"receive" after any pre-written text has been sent.

shift F1:

This switches the baud rate. When a change is required in  the

de df1gw

PAKHOST                      - page 8 -

baud  rate  an  "X"  will  be introduced. The Pakratt uses the

command "XBAUD". The effects on the shift  condition  will  be

made automatically.

shift F2 - F4:

These  will  enable  the parameters to be altered (see rx win-

dow).

shift F5:

This switches to "letter"-receive.

Terminal Window Amtor:

Title Receive Window:

This  will  show  the  amtor conditions on the top line of the

mode selected: AM=AMTOR : AL=AMTOR LISTEN : FE=FEC : NA=NAVTEX

: TV=TDM (MOORE).

Send Window:

The  outgoing text will be sent when either the "space bar" or

"return key" has been pressed. As long as  the  word  has  not

been completed it is able to be corrected.

F10:

This  is the toggle to send key.This will be interogated for a

"SEL CALL" and if none is found the  Pakratt  will  switch  to

"FEC" mode.

CTRL D:

This will terminate the connection (link down)  with  ARQ.(ARQ

+? to the This will toggle into Receive when in "FEC" mode.

siftt F1:

This will toggle the received modes AL:AM:NA:TV.

shift F2:

This will send an automatic change over during an existing ARQ

connection.

shift F3 and F4:

This  enables the shift condition of the receive and send sig-

nal to be interchanged.

shift F5:

This switches to "letter"-receive.

Terminal Window CW:

Title Receive Window:

This displays the sending speed in Words per Minute.

Send Window:

The outgoing text will be sent when either the "space bar"  or

"return  key"  has  been  pressed. As long as the word has not

been completed it is able to be corrected.  WARNING:  "Return"

will add an equal sign in the text.

Ctrl D:

This will be inserted in the normal text and  will  switch  to

receive after any pre-written text has been sent.

shift F1:

This will alter the sending speed.

shift F2:

This will freeze the immediate receiving speed.

Terminal Window Pactor:

de df1gw

PAKHOST                      - page 9 -

Title Receive Window:

This will show the pactor conditions on  the  top  line   left

(while entering this window): Pt = Pactor, PN = Pactor listen,

PD = Pactor transmit, PG = Pactor connected.

Send Window:

The  outgoing text will be sent when either the "space bar" or

"return key" has been pressed. As long as  the  word  has  not

been completed it is able to be corrected.

F10:

This is the toggle to send key. This will be interogated for a

"Partner Call" and if none is found the Pakratt will switch to

"FEC" mode. After a disconnect type only "!" to reconnect with

dx-parameters.

CTRL D:

This will switch to receive while unprotocolled transmitting.

CTRL Z:

This will switch to receive while connected in pactor.

shift F1:

This will toggle the modes "Pt (Pactor)" and "PN  (Pactor  li-

sten)".

shift F2:

This  will  send  an  automatic change over during an existing

Pactor-connection.

shift F3:

switches between automatic-baud (200 or 100)  and  fixed  baud

(100).

shift F4:

toggles the "Huffmann-compression".

HELP MENU:

This has been arranged so as to conceal the seldom used  parts

of  the program and  in  order  to  give  every  possiblity to

operate peripherals. The author will GUARANTEE  that  Crashes,

Bombs  etc  with all peripherals are easily brought about; but

mostly these are caused when using the V52 or similar.  THERE-

FORE.....TAKE CARE.

MDCheck:

This will enable the Mailbox commands to be executed so far as

the pakratt firmware and hardware to date. The MDcheck is able

to  be programmed but unfortunately HOST MODE does not support

it. The calling of this part of the program will activate  the

Mailbox  during which the packet radio monitor will switch off

and eventually the stored contents  of  the  monitor  will  be

written to the file "GULP.ASC". Also here with shift "F10" you

are able to call for the printer or disk drive. One leaves the

MDcheck  with "UNDO". The Mailbox can be switched on or off in

the old parameter window using shift "F1" and  the  correspon-

ding indicator will occur in the upper title line.

F-Keys:

Here  the  Standard Texts (seperated for each operating mode),

the special texts, the  standard  connects  and  the  standard

paths  can  be  edited (this can also be done using a standard

WP). Control characters are allowed with the  Standard  Texts.

de df1gw

PAKHOST                     - page 10 -

Those  of  note  are:  CTRL D for the receive switching of the

Pakratt and CTRL W after the text no Cr should be issued.  The

standard  texts  can  be created with multiple lines, for this

purpose 100 lines are allowed but not  over  written  and  for

this  reason  there  will  be  requested the line numbers. All

other parameters are in single lines.

Although its seems complex it is not: please try it!

Following on from any alteration, Pakhost  asks  whether  this

should be secured in the file FTAST.DAT, with which it will be

stored. The old file will remain on disk as FTAST.DUP.

change parameters:

For  editing  the  file PH_PARA.DAT. You will get a list with

PK232-commands and there contents. You may change the parame-

ters  of the 'passive'-list after your own opinion. Following

will happen: The PAKHOST sets your Pakratt to this parameters

while leaving the program. Just before it will write the cor-

rosponding  active parameters to the file PH_PARA.DAT. At the

next start of PAKHOST, your PK232 will be  resetted  to  this

'active'-parameters.

Choose the parameter with the code on the left side (you must

use capital or low letters!).

If  you don't want the parameter to change, please delete the

contents of it in the 'passive'-list. i.E.  if  'passive'  is

empty, the parameter will not be changed (and vice versa).

Attention: the 'passive'-parameters will be  set  your  PK232

also using the procedure 'init' (see below).

init:

Initialises all the important parameters, which  will  be  re-

quired  for the correct operation of Pakhost (page1), sets the

userspecified parameters from PH_PARA.DAT (page2) and shows on

a third page all read only parameters.

divers:

From  here individual parameters can be altered, which are ef-

fective in all modes. The parameter " CUSTOM" can be installed

very simply from here and for this a specialised  window  will

be opened in which the Bit for Bit can be altered. Short texts

help to distinguish the meaning of the Bits.

date:

This is used to set/alter the Pakratt and/or Atari date

host mode:

This will switch the Pakratt into HOST MODE..crash danger !!!

normal mode:

This  will  switch  the Pakratt into NORMAL MODE..crash danger

!!!

PRG.START

From here an external program can be run(ie  text  editor).The

program  (pakhost) will have provided 250k ram at the disposal

of the external program as long as if with "FTAST" under  "DI-

VERSE" operation another value has not been allocated.For most

programs the allowance is adequate with a 1MEG ST.

ASCII LISTEN

This sub-program will allow you to look at text files and  af-

ter  each page you will be asked whether you wish to read more

or abort.

de df1gw

PAKHOST                     - page 11 -

PAKHOST

This returns the program to the central window

LOG BOOK

A  small routine in order to open the log book file can be ob-

tained from the main window with "F9".The description  of  the

file  is at the end of this guide.The file allows itself to be

loaded from current databanks.(to that  end  the  Field  "EOL"

serves  especially as it contains no data but just "end of li-

ne".By this means the Dbase command "append" sdf from RUFZ.DAT

is possible).

Help:

for the callinput. (same is <F9> or <shift F9>.)

With the issue of "L" the automatic packet radio log is  swit-

ched  on  and  all  connects  and disconnects will be recorded

along with the time and date from the atari clock.

If the logbook is called from a terminal window using Shift F9

then  identical fields will be requested.Only with a call from

the terminal window is there no more room for  field  names,so

please  as  before  Practise.A few input checks will be reque-

sted;ie you may only put in a frequency with a decimal point.

The logbook will require a fairly long  initialisation  period

when  there  is a lot of data in it.On these grounds it is not

automatic but will be loaded with the first call  from  F9.The

loading proceedure will be displayed graphically and there for

involves  no  saving  of time when it arrives and Pakhost must

page through the callsign list.Never the less the partner call

saving with Shift F9.

In  the  upper  title  window "LOG" will be displayed when the

logbook is active. With the activation  of  the  autolog  then

"auto-log" will be displayed but only in packet radio and pac-

tor. The files RUFZ.DAT and LOG.DAT will be secured on disk at

all times when leaving the large logbook or the PAKHOST.PRG.

DISK AND PRINTER OPERATION:

This window is obtained by pressing Shift F10 from the  termi-

nal window or from the Mail Drop Check.

After it is called it will indicate the existing lines of  the

Ram  Buffer which can be called using "Clr/Home". The size of-

the ram buffer adjusts itself according to the respective con-

figuration  and free ram space. If more than 50k free space is

available then this will be displayed, otherwise the free spa-

ce  remaining.  If  the  program is not started from drive "A"

this test does not take place (the TOS-routine for this opara-

tion does spend to much time.).

If a printer or disk protocol is employed then the  length  of

the protocol file in bytes is displayed.

Return:

returns to terminal operation

INSERT

A file will be opened and sent on the last  installed  channel

to  the  radio parameter ie packet;cw;etc.If the file "tx.asc"

is on hand it will be defaulted to this.The choice of files is

de df1gw

PAKHOST                     - page 12 -

available from a selection box.Subsequently the  program  will

request  whether  or not the file should be ended with a "ctrl

Z".The latter is very useful if long files are  written  to  a

mailbox.the  end  of  the file transfer then requires personal

monitoring.

CLR/HOME

The text saver of the last free channel  will  be  written  to

disk;no  other  channel  will be affected.Also this key can be

used to clear the the text ram;(contents will be  saved  under

default  names).Only the received text and the associated con-

nect messages will be saved.If a file "RX.ASC" is available it

will be defaulted to this.

DELETE

This will clear a file.A confirmation request will be issued.

RIGHT ARROW

The disk protocol for this channel will be installed.This pro-

tocol adapts itself to very long texts which find no more room

in the normal ram buffer.It will be written with ,what will be

received on the already active channel(also the connect messa-

ge).Except  in  packet radio where the transmitted text(echoed

data) will also be saved.If the protocol is active it will  be

displayed in the heading of the master window.

LEFT ARROW

The disk protocol will be removed

DOWN ARROW

The print protocol of the active channel will be installed and

everything  received on that channel will be printed including

the connect messages.Except in packet radio where  the  trans-

mitted  text will also be printed.The "umlaut" modifier is not

active here.The printer obtains only the received  ASCII  cha-

racters.If  the protocol is active it will be displayed in the

master window.

UP ARROW

The print protocol will be removed

F5

This will send a form feed to the printer.

REMOTE CONTROL:

This part of the program is only for test operation. There are

distinctly  better remote possibilities in other programs. But

if you should miss one for one of your  important  diversions,

then i am agreeable to consider your wishes.

If the remote control is active (rem in the central window) an

the atari is switched on,then the connected station can opera

te the following commands:

//B        the Atari rings

//I        ATARI WILL SEND FILE : INFOFILE.PKH

//H        ATARI WILL SEND FILE : HELPFILE.PKH

//M        THE MH LIST WILL BE SENT

//E TEXT    "TEXT" WILL BE RETURNED

//D        THE DIRECTORY CONTENTS OF THE DRIVE WILL BE SENT

//D PATH   THE DIRECTORY CONTENTS OF THE PATH WILL BE SENT

//R FILE   THE CONTENTS OF THE "FILE" WILL BE SENT

de df1gw

PAKHOST                     - page 13 -

//W FILE   THE FOLLOWING WILL BE WRITTEN IN THE FILE "XXX". IF

A FILE EXISTS WITH THE SAME NAME, THE NEW TEXT WILL

BE APPENDED.

//Z        THE FILE OPENED WITH //W WILL BE CLOSED

//Q        THE CONNECTION WILL BE ENDED

With the remote control switched on the text "REM" appears  as

a  warning  for  the operator in the upper half of the receive

window.

DESCRIPTION OF FILES

FILES

The following files must be on the working disk:

PAKHOST.PRG     The main program

PAKHOST.RSC     The GEM resource file

DESKTOP.INF     This is important for the parameter  installa-

tion especially with newer  TOS  versions  and

hardware   handshake.Use either Desktop.Inf or

Newdesk.Inf

PAKHOST.DOC      This  should  not  be on the working disk but

must accompany the permitted  distribution  of

the  program. This will ease the burden on the

author.

The following files will be created either by Choice;the  ope-

rator; or the program itself.

FTAST.DAT       The standard texts for Fkey allocation

FTAST.DUP       FTAST.DAT backup

PH_PARA.DAT     parameter for automatic change while starting

or leaving PAKHOST

PH_PARA.DUP    PH_PARA.DAT Sicherung

RUFZ.DAT        Logbook file:Data of the QSO partner

LOGB.DAT        Logbook file:Data of the QSO'S

GULP.ASC        This will be created by the program  and  will

secure the text which will possibly be contai-

ned in the ram of the Pakratt.

HELPFILE.PKH    Help text for remote operation.

INFOFILE.PKH    Information text for  remote  operation.  This

file will contain personal  data  and  can  be

edited  using  a  text editor before the first

operation. Both of the *.PKH files must be  in

the root directory.

XXXXXXXX.XXX    Diverse  files,which when saved are able to be

self generated and named.

ACCESSORY PROGRAMS

EMULATOR.ACC   Atari V52 emulator

FTAST.EXP     Example file for FTAST.DAT

HELPFUL FILES

RX.ASC      This will be used to save the received text if  it

is on hand.

TX.ASC       Text to be transmitted will be looked for in this

de df1gw

PAKHOST                     - page 14 -

file first.

FTAST.DAT FILE DESCRIPTION

The file will be created by the PAKHOST.PRG and can be  edited

from  the "HELP MENU"(called by "help" from the central window

and clicked from F-TAST under Pakratt).From time to  time  the

old  version from FTAST.DAT will be backed up as FTAST.DUP.The

file can also be created using a  text  editor.This  must  end

each  line  with  a combination CR/LF and only save pure ASCII

text(eg Tempus).A standard text may occupy a  maximum  of  100

lines.

The file FTAST.EXP is "my FTAST.DAT" and  should  provide  for

you  an  example.To  use  it  ,it  will  require to be renamed

FTAST.DAT and will need to be edited immediately.

The  following  Control  Characters stand at the begining of a

line.

*CO*  Commentry;will not be actioned.Also  all  characters  in

the same line from *XX* will be ignored.

*0X*  Previously condensed

*03*   Ram  reservation  for  the program.No entry will assume

250,000

*04*  PK232 connected to MODEM1 or MODEM2

*14*  The text with which packet radio will be called by F4

*25*  The text with which RTTY will be called by F5.The  first

figure also stands for the mode.1 packet radio;2 rtty; 3

ascii;4  amtor;5  cw.The  second figure is the number of

the function key which should be allocated(1  to  8).The

function  key  9  should  not be allocated as it will be

used from the program as quicker intersaving of the con-

nected  call.  A Standard text will alway be sent with a

CR. You can also draft multi-line texts.For cases  where

the  CR  should  not  be  appended,please attach a CTRL-

W(EOB). Please look at the example in FTAST.EXP

*7X*  Standard Connects

*70*  F10

*71*  F1-F8

*79*  F9

*8X*  Standard Paths,X as with *7X*

*CL*  Ends the file and MAY NOT BE OMITTED

RUFZ.DAT (C/SIGN)   Random access file:Record size =90

-----------------   ----------------------------------

RUFZ    . .    8 Bytes

VORNAME  (forename)  10 Bytes

WOHNORT  (home town) 20 Bytes

QTH          6 Bytes

DOK          4 Bytes

REM          40 Bytes

MARKR         1 Byte:Y or N

EOL          2 Bytes:CR/LF

LOGB.DAT        Random access file:Record size=66

--------        ----------------------------------

de df1gw

PAKHOST                     - page 15 -

RUFZP         3 Bytes

RUFZ          8 Bytes

RUFZS         4 Bytes

REMR          1 Byte:Blank,P,M,A (or what else)

DATUM   (date)   8 Bytes: XX.XX.XX

QTRB          5 bytes: XX.XX

QTRE          5 Bytes: XX.XX

QRG          8 Bytes: XXXX.XXX

MODEQ         4 Bytes:PR;RTTY;ASCII;AMTOR;FM;SSB;AM;CW;FAX

RSTO          3 Bytes: XXX

RSTI          3 Bytes: XXX

REML          10 Bytes

QSLPSE         1 Byte:Y or N

QSLO          1 Byte:Y or N

QSLI          1 Byte:Y or N

MARKL         1 Byte:Y or N

EOL          2 Bytes:CR/LF

Index to Pages:

AMTOR              1,5,6,8,14,16

ASCII              1,2,7,10,12,14,16

atari              1,2,3,4,6,10,11,12,13,16

BAUDOT             1,4,7,16

change             1,2,3,4,7,8,9,10,13

clock              3,11,16

CText              3,4,16

cursor             6,16

CUSTOM"            10,16

CW                 1,6,8,14,16

DESKTOP            2,13,16

FTAST              2,3,4,7,10,13,14,16

handshake          2,13,16

HELP"              3,7,14,16

HELPFILE           12,13,16

INFOFILE           12,13,16

INSTALLATION       2,3,4,7,13,16

LOG                3,6,11,16

Mailbox            1,3,4,5,6,9,12,16

MAIN               3,6,7,11,13,16

MODEM2'-connector  2

NAVTEX             1,16

PACKET             1,4,5,6,7,9,11,12,14,16

PACTOR             1,5,6,8,9,11,16

PAKHOST            1,2,3,4,6,7,10,11,13,14,16

PH_PARA            10,13

REMOTE             4,6,12,13,16

TDM                1,16

de df1gw