PICTURES FROM SPACE

IMPORTANT INFORMATION

PLEASE COPY THE PROGRAMS TO YOUR WORKING DISKS BEFORE RUNNING THEM

LIABILITY OF THE AUTHOR

This  software  has  been  developed  for  the   Maplin

Electronics  MAPSAT weather satellite receiving  system

project.  It  offers the opportunity for ST  owners  at

hobby   or  professional  level  to   explore   weather

satellites  at low cost (a couple of  hundred  pounds),

while  not compromising on the features to be found  in

systems costing very considerably more.

The  author has used  the equipment  described  without

problem for over five years.  No liability is  accepted

by  the  author for any consequences arising  from  the

connection of any device to your computer.  You  should

contact your hardware supplier if in doubt.

MAPLIN ELECTRONICS PLC.

catalogue    P.O.  Box 3,  Rayleigh,  Essex  SS6  8LR

England.   Telephone (0702) 552911

or from W.H. Smiths, newsagents

The author has no connection with the above, however he

would  be pleased to assist interested  persons  living

outside  the U.K.  in ordering the necessary  hardware,

and dealing with any matters arising.  (Return  postage

appreciated).

COPYRIGHT

(c)1990  Leslie  John  Kaye,  Fieldvale,  Park  Lane,

Snitterfield, Warwickshire CV37 OLS.

This  manual and software may be copied subject to  the

following conditions:

1) No  charge  is  made other than a  nominal  fee  for

duplicating and related expenses.

2) All the following files are included unaltered.

PFSSW.PRG               PFSORBIT.PRG

PFS.DTA                 KEPLERS<FOLDER>

PSMANUAL.DOC            PFSREAD.ME

REGISTRATION

The accompanying software is a full working version  of

the PFS software that was available commercially  until

early 1990.  As such it may well fulfill your  decoding

requirements.  If so, please register as a user wth the

author.  The 12-50 registration fee covers the project

advice service,  current satellite information, sources

of information, and current product information.

1990 saw a major re-write of the software. The decoding

speed  was  more  than doubled,  user  control  of  the

decoding  speed,  choice  of  file  sizes,  and  colour

palettes   (to  take  advantage  of  the   STE   colour

capability)  was  provided.  These and many  other  new

features may tempt you to purchase the current  version

of  PFS  software.  In  addition  to  the  benefits  of

registration  mentioned above,  you will  receive  your

Manual in booklet form.  The price of the software  and

manual is 25-95 (+ 2-50 postage outside Europe).

The  prices are pounds(-pence) Sterling and fixed until

1st. January, 1992  after which  5% per year should  be

added.

PROJECT ADVICE SERVICE

A  hardware get you going service is offered by  Maplin

for the MAPSAT project.  The author is pleased to offer

any  reasonable  assistance  or  advice  to  Registered

Owners. A stamped addressed envelope should be enclosed

with queries. No miracles are promised!

UPDATES

PFSSW.PRG is under regular review,  as are the  various

utilities.   Registered  Owners  will  be  notified  of

significant  developments,  which will be available  at

modest cost.

PICTURES FROM SPACE

USER MANUAL FOR THE ATARI ST

CONTENTS

1.0  Introduction

1.1  Welcome to Pictures from Space software

1.2  Remote Imaging Group

2.0  Satellite orbit prediction

2.1  Satellite selection

2.2  Recent satellite position

2.3  Printing the forecast

3.0  Decoding sequence

3.1  Scan screen

3.2  Decode trouble-shooting

4.0  Picture processing

4.1  Data and position

4.2  Titling an image

4.3  Processing options

4.4  Grey level adjustment

4.5  Zoom

4.6  The colour palettes

4.7  North to South

5.0  File operations

5.1  Load

5.2  Save

5.3  Save Degas

5.4  Print Degas

6.0  User programs

Appendix  A.1  Hardware

A.2  Project construction

A.3  Interfacing

1.0      INTRODUCTION

There are now many satellites in orbit, using the WEFAX

standard to transmit images of the Earth. The main uses

of  these images are for weather forecasting,  and  the

study of oceans and the atmosphere.

The WEFAX images comprise individual lines of a quarter

to  three seconds in length,  sent by modulating a  2.4

khz carrier wave.  A tone marks the start of each line.

By putting the lines one above the other,  an image  is

formed.  In  the  case of low orbiting  satellites  the

movement of the satellite increments the line position,

so  a continuous image is received while the  satellite

is  in  radio  range.   The  geostationary   satellites

transmit individual frames of 800 lines.

To receive low orbiting satellites,  you need a  137mhz

receiver,  preferably purpose designed,  and a  crossed

dipole  aerial.  This  receiver  may  be  used  with  a

frequency downconverter and,  preferably, a dish aerial

for   reception   of  the   geostationary   satellites.

Alternatively  a 1600mhz receiver can be  used.  It  is

more convenient to use a simple cassette tape  recorder

to  record  the transmissions.  This also  removes  the

problem of radio "noise" created by the computer,  when

decoding  direct.   A  decoder  converts  the  analogue

signals  received into numbers for  your  computer.  It

also generates the start of line synchronisation pulse.

1.1      Welcome to Pictures from Space software.

If  you  are  about to  construct  the  Maplin  weather

satellite decoding system project, I hope you will find

the  comments in appendix A  useful.  In any  case  the

interface  cable  must be wired as  described  for  the

program to work with the decoder.

The   Maplin  Magazine  articles  supplied   with   the

project contain much useful information. This manual is

intended  to  supplement  the  articles  for  Atari  ST

owners.

PFSSW.PRG  and  the  standard data  file  occupy  about

350,000 bytes, so will fit into most STs  provided that

other large programs are not also resident.The software

has  been  tested with the Enhanced ST  (STE)  to  take

advantage of the increased range of colours  available.

It works in low resolution only (colour monitor or TV).

Screen and raw data files can be saved to disk,  as  it

is  envisaged that most users will want  to  experiment

with their own programs.

PFSORBIT.PRG  works in any screen resolution.  It gives

a forecast of usable  satellite passes  for a period of

about three months.

1.2      Remote Imaging Group

This  group was formed in 1985 to promote  and  further

interest in weather satellite watching,  and to  liaise

with the various official bodies  involved.  Membership

benefits include a quarterly newsletter,  and automatic

permission  to receive satellite  transmissions.  There

are now over 1,000 members.  Current membership details

are  included  in the information  sent  to  Registered

Users.

Some   radio   magazines   carry   weather    satellite

information. Short Wave magazine has an "Info in Orbit"

regular feature, at the time of writing.

2.0      SATELLITE PREDICTION

Low  orbiting satellites are in close to polar  orbits,

i.e. fly over the North pole, down over the South pole,

then  northwards  again.  The  orbit  takes  about  100

minutes, in which time the Earth spins about 25 degrees

so  that the next satellite pass seems to be  about  25

degrees further West to an observer.

The  American  Tiros satellites (NOAAs  9,  10  and  11

currently)  are in sun synchronous orbits so that  they

can  be  observed  at  about  the  same  time  of  day,

throughout the year.  Russian Meteor,  Okean and Cosmos

satellites appear hours earlier as the months pass.

PFSORBIT.PRG may be used to predict any Earth satellite

from  the "Kepler Elements" which describe  the  orbit.

These Elements are published by various  sources,  such

as  the Remote Imaging  Group  Newsletter.  Information

should be no more than 1 year old,  and preferably much

less.

These Elements are

EPOCH  (year day.fraction of day) when a measurement

was taken at the Equator.

INCLINATION  angle of orbit (90 = due North)

RIGHT ASCENTION  OF THE ASCENDING NODE  position  of

the satellite on the Equator compared  with  a fixed

point in Space (First Point of Aries).

ECCENTRICITY  deviation of the orbit from a circle.

MEAN MOTION  approximately the number of  orbits per

day.

ACCELERATION daily increase in Mean Motion caused by

the atmospheric drag on the satellite.

Other Elements quoted are not required for our purpose.

Show and print options give about three months  forward

prediction  of satellite times overhead  the  observers

latitude.  The satellites may be heared 5 to 10 minutes

before until 5 to 10 minutes after the predicted  time.

Small  inaccuracies will appear for  various  practical

reasons,  most  significantly the  acceleration  (orbit

decay) varies unpredictably.  It is therefore desirable

to  record observations as accurately as  possible,  to

monitor the prediction and obtain a starting point  for

the  next three month prediction.  The program  may  be

used with any screen resolution.

2.1      Satellite selection

After double clicking on PFSORBIT.PRG,  and pressing  a

key to acknowledge the copyright information,  you  are

asked  to enter the name of a  satellite.  The  program

takes the first 8 alpha-numerical characters  (starting

at  the first alpha character) to make up a file  name.

It  then  checks the KEPLERS folder to see if  data  is

already on file. You may overwrite the information with

more recent data if you have it.The program will create

a KEPLERS folder if there isn't one on the disk. Do not

delete the contents of the folder otherwise the program

will crash. (delete the folder as well, if deleting the

contents).

Be careful with file names.  For example "Meteor  2.18"

and "Meteor 2.19" will both give the same file name  of

"METEOR21" and therefore clash. "Met 2.18" and "MET218"

would also create the same file name as is desired.

The  satellite  position  at the  time  of  the  Kepler

measurement  is  then calculated (Northbound  over  the

Equator)  and adjusted to give the time  and  longitude

of  the satellite at the observer's latitude  (e.g.  52

North for U.K. observers.).

2.2      Recent satellite position.

You  are then asked if you wish to enter a more  recent

observation.

Most  sources  of orbit data give  times  and  position

Northbound  over the Equator.  (Kepler  elements,  also

NOAA information from Weatherwatch UK,  telephone  025-

683-448  evenings  holidays and  weekends  only.).  The

program  requires the overhead time,  so  add  e.g.  15

minutes  and  4  degrees West  approx.  for  52  North.

(Subtract  if  your  position  is  South).  Sources  of

information   also   generally   use   Universal   Time

(Greenwich Mean Time). It is more convenient to convert

to your local time (e.g.  British Summer Time) for your

forecast.

2.3      Printing the forecast

The  forecast may be printed out,  as well as shown  on

the screen.  The printer control codes are kept  simple

so as to suit most printers.  Information given are day

and   month,   time,   longitude  and  whether    North

to South, or South to North. You may pause the forecast

by pressing a key.

3.0      DECODING SEQUENCE

The hardware instructions describe how the transmission

can  be decoded as it is being received.  The  problems

include;  the considerable radio interference caused by

the computer,  monitor and printer. Also the difficulty

in  fitting  a  twelve  minute  transmission  into  the

computer.  It is more convenient to record on tape, and

decode afterwards. Test your decoder as far as possible

in  accordance with the instructions before  connecting

to your computer.

A satellite  pass should be recorded. The satellite may

be  picked up 5 to 10 minutes before passing  overhead.

Taking a note of the exact time of turning on the  tape

recorder  will  allow  position/time  location  of  the

satellite to be identified from the decoded image.

Do   not   insert   or  remove  any   plugs   to   your

computer, while powered up.

Select   "decode"  from  the  menu  bar.   The   screen

clears other  than the prompt notes.  Start  your  tape

playback with the system configured as described in the

instructions.  The  receiver  should  have  "tape  out"

selected.  Volume may be turned low or off.  The  three

level  controls  on the decoder may  initially  be  set

about  1/4  up.  The  optimum  settings  are  found  by

experiment.

3.1      Scan screen

A  coloured  scan  line should  start  working  up  the

screen.  After a short time one edge of a NOAA  picture

should  be seen as a vertical border  with  rectangular

blocks on one side,  and a horizontal line marking each

minute,  on the other.  The input level on the  decoder

should  be  adjusted so that much of one  side  of  the

border is just white, and the other dark blue or black.

The image on one side of the border will be darker than

the other.  This is the optical image.  Use the decoder

slip  control  to shift the border to the side  of  the

screen.  Colours range between low red, and light blue.

With an infra red image,  the input level adjustment is

very  fine.  the  levels should be adjusted to  give  a

minimum of dark red,  and light blue.  Press the  space

bar  to  restart  the scan from  the  top  after  these

adjustments.

The computer will now continue to scan,  restarting  at

the top of the screen each time the bottom is  reached,

progressively  overwriting the  previous  image  as  it

goes.

The  image  may be seen to be taller than it  is  wide.

This can be adjusted in the picture processing mode.

The scan can be restarted at the bottom by pressing the

space bar.  If any other key is  pressed,  the computer

will not ackowedge (other than the key click) but  will

leave  scan mode when either the  top of the screen  is

reached, or the space bar is next pressed.

Note that two scan lines are displayed per screen  line

to  allow 400 data lines to be saved to  memory.

On  leaving scan mode,  a low resolution image  of  the

whole data file will be shown.

3.2      Decode trouble-shooting.

Problem                  Possible cause

No scan line when tape   No line sync pulse received on

playing, and decoder     Data line 0. Hardware problem.

switched to tape out.    Check all cables for a start.

Image wavers sideways    Automatic line synchronisation

not fitted/not working.  Check

switch setting, and hardware.

Wavering  of  the  left  hand

edge  may be caused  by  dirty

tape heads.

Image diagonal           Lines per second slightly out.

Check switch and diode board.

Poor or rubbish image/   Lines per second significantly

Screen lines obviously   out. Or if you can see a clean

out of step with         edge of frame, the picture may

audible pulse            taken in poor light but OK.

4.0      PICTURE PROCESSING

When  an  image is displayed after exiting  scan  mode,

exiting option 4.3 (image processing) or loading a data

file from disk, options 4.1, 4.2 and 4.3 are available.

4.1      Data and position

Pressing  the right mouse button shows the mouse  cross

hair.  In  the top right corner of the screen is  shown

the data file value under the mouse,  together with the

x and y screen position.  Measuring the screen distance

between known geographical points allows a rough  scale

to be calculated,  but note that the satellite image is

distorted at the edges.  Obtaining the data values  for

the  darkest and lightest areas helps find the  optimum

black and white settings (refer 4.3).

4.2      Titling an image

Press  either [Shift]  key to show the text  cursor  in

the bottom left position. Type in a title to the image.

If  you need more than one line,  enter [Control]J  and

[Control]M to start the next line.  Press [Return] when

finished.  Save  Degas immediately,  otherwise  further

image processing will overwrite the title.

4.3      Processing options

Pressing  the  left mouse button shows  the  menu  bar,

together  with the main image processing options  in  a

window style box.

The  window sliders allow the desired part of the  data

file to be viewed. These vary with the zoom options.

4.4      Grey level adjustment

If  the surface of the Earth is visible it can  usually

be  identified  by coastlines which  appear  as  smooth

edges in comparison with the ragged cloud outlines.

If  one  side  of the image is  light,  and  the  other

dark,  a morning or evening view is probably shown. The

black and white level buttons may be experimented  with

to  explore  your image.  If your clouds  are  normally

green or light blue,   the image should be improved  by

adjusting  the  levels on the  decoder  and  re-running

"decode" from the menu.  Otherwise these are best  left

alone, once satisfactory results are obtained.

Adjustment  of the black and white values sets the data

values corresponding with colours 0 and 15.

4.5      Zoom

Lo  resolution  averages four data values to make  each

screen pixel.

To  explore an area of the image in hi resolution  (4x)

the area should be indicated by framing with the window

slider bars, and "hi" resolution selected.

Wide  mode gives a double width correction in  lo or hi

resolution. It is toggled by selecting the [<>] box.

Clicking the "OK" box will re-draw the  adjusted image.

The raw data file is not affected.

4.6      The colour palettes

Three colour palettes may be quickly selected  from the

menu.  The ST is only capable of showing 8 grey levels,

and the STE 16 grey levels.

On  running the program, the default colour palette  is

set  with  low red,  black,  three blues,  two  greens,

greys  to white and light blue.  This usually  gives  a

satisfactory result with optical images of low orbiting

satellites.  It should be remembered that colours shown

are false, and that for example, green clouds are quite

possible!

The  luminance palette should be used with a black  and

white  T.V.  or with the colour control on your  colour

T.V.  or monitor turned low.  With this palette 16 grey

levels  are  achieved by using the  brightness  of  the

settings and ignoring the colours.

A 8 grey palette (16 with STE) is also provided. It  is

preferred for infra red and poor optical images.

Whichever  palette  you choose,  only  the  display  is

affected.  The  raw  data can be  saved  to  disk,  and

reloaded with another palette if desired.

4.7      North to South

Where   the  satellite  has  descended  from  North  to

South,  North  will  be at the bottom  of  the  screen.

"South to North" on the menu bar should be selected  to

invert the data file to give the usual view with  North

at the top.  This is the only option which modifies the

data file.

5.0      FILE OPERATIONS

The  file operations are available from the  menu  bar.

No  "delete"  file option has been included  so  as  to

prevent  accidental deletion of files.  Unwanted  files

should be dragged to the trash can from desktop.

5.1      Load

Previously  saved data  files may be loaded from  disk.

After  this  long  operation  the  picture   processing

sequence is entered.

5.2      Save

The current  data  file may be saved to disk.  Only one

file will fit per side of disk.  The raw data from  the

decoder is saved without being altered,  other than  by

the  "South to North" function.  File names should  end

with .DTA.

5.3      Save Degas

The  current processed image (lo or hi  resolution)  is

saved  as a Degas PI1 file for use with that  excellent

program, and other compatible software.

5.4      Print Degas

Any  previously saved Degas PI1 file may be printed  on

an Epson compatible printer.  For best results, process

the image on the Grey scale palette (for near-wysiwyg!)

to give the full palette range of contrast. For non PFS

PI1s, colour palettes should be set in increasing order

of luminance.  The quality of the image would  probably

not justify rushing out to buy a printer if you do  not

already have one.

6.0      USER PROGRAMS

You  may  wish to  write your own programs  to  further

analyse or process the data.

The  data  is   filed in bytes using the  even  numbers

between 0 (dark) and 254 (light).  No other information

(e.g.  header,  line markers etc.) is in the file.  The

data bytes are stored as follows:

byte   byte   byte   byte          byte   byte

data |     1|     2|     3|     4| .... |255999|256000|

file |  word      1|  word      2|      |  word 128000|

This  comprises 640 bytes by 400 lines starting at  the

top left (North West) corner.

The  data needs to be converted to screen file  format

to display an image.

Degas compatible files comprise:

Word   1   = 0    = low resolution file

Words  2 to 17    = colour palette

Words 18 to 16017 = screen image

APPENDIX A

A.1      Hardware

Pictures  From Space is believed to work on any  ST  in

low  resolution,   with  T.V.  or  colour  monitor.  In

addition an APT receiver and decoder is required.

If  the  Maplin  Electronics Mapsat project  is  to  be

constructed,   the   receiver,   aerial  and   decoder,

including  optional  items  are  required  to   receive

pictures from low orbiting satellites. These are listed

on pages 319 and 320 of the 1991 catalogue.  Instead of

the   decoder  interface  cable,   the  following   are

required:

25 way D plug           YQ48C

25 way D cover (2 OFF)  FP29G

25 way D  socket        YQ49D

15  way multicore cable XR28F

Various  small  tools are needed,  including  a  decent

soldering  iron  of about 15 watts,  having a  tip  not

larger than 2.5 mm..  Solder not thicker than 22 swg is

strongly recommended. A wire cutting shear is needed in

addition to a stripping and cutting tool.  A  voltmeter

is essential to check the decoder construction,  but  a

multimeter is more useful.  The ohmmeter can be used as

a continuity tester to check that solder has not run to

a nearby track.

A.2       Project construction

The  Mapsat project looks rather daunting as  its  many

boxes  arrive  in the post.  Although it  will  take  a

number of evenings to complete,  it is not a  difficult

task even for those new to a soldering iron. A guide to

components  and  construction  is  included  with   the

instructions.   If   you  have  never  constructed   an

electronics project before,  it is advisable to order a

small  project  just  to ensure  that  you  can  solder

tidily.

The  receiver is a good starting point,  as the PCB  is

already built, and satellites can be recorded while the

decoder  is constructed.  The hardest part is the  case

which was designed by an idiot.  All the fancy bits  go

to  the  front.  Make sure you understand how  it  goes

together before drilling holes.  Keep metal filings and

swarf away from the electronics.  The PCB (as with  the

decoder)  should be nearly flush with the back  of  the

case.  Before  drilling the holes for the plugs in  the

back of the case,  check that they will be in line with

the  sockets  on the PCB.  Make the holes  oversize  to

avoid  a  struggle with the plugs.  Take your  time  to

ensure that wiring is done correctly.

The  aerial is supplied with a black plastic liquid  to

fill  the  box which holds the metal  rods.  It  hardly

covers the bottom, and should be bulked up by mixing in

a suitable solid, such as fibreglass filler, or some of

the millions of plastic packing pellets  provided.  The

holes  through which the rods are put,  must be  sealed

with glue or similar to prevent the black liquid oozing

out before it sets.

The project now becomes interesting as the merry clip -

clop  of the NOAA's,  or melancholy quank quank of  the

Russian satellites can be heard.

The decoder construction is easier than it  looks.  Lay

out  all the components in the same order as listed  in

the  instructions.  Make sure that you have  identified

them all.  Start with the resistors and take each group

of identical components in turn, in methodical fashion,

and  solder  in  place.   Ensure  that   diodes,   some

capacitors,  ICs etc.  are the right way round. See how

consistent you can be with the soldering,  keep the tip

of the soldering iron clean by wiping on a wet  sponge,

keep  it  hot and work  quickly  and  accurately.  Some

components require the solder to run through the  board

to connect top and bottom. Be very careful with these.

A.3      Interfacing

The  25  pin  plug and socket should  be  connected  as

follows:

Decoder socket       Computer plug

(pins for Mapsat)     (printer port)

pin                 pin

data 0      25       not connected

data 1      12                   3

data 2      24                   4

data 3      11                   5

data 4      16                   6

data 5      17                   7

data 6      18                   8

data 7      19                   9

SYNC        22       data 0      2

EOC         23       not connected

ground    3-10       ground  18-25

The  pin  numbers marked on the plug and socket  should

be identified to avoid connecting the wrong pins.