ST Color Palette

Displays the ST's 512 colors

by Tom Hudson

Most new Atari 520ST owners, if they're like me, want to see what kind of fancy

graphic "tricks" their computer can do. Here's a short program, written in

Digital Research C and 68000 assembly language, which displays all the ST's 512

colors on your computer screen at once. The ST Color Palette (STCP) also allows

you to determine the color register settings needed to use that color in your

own programs, by simply moving the mouse. If you don't have a C compiler or

assembler, don't worry - the compiled program will be available on the ANALOG

Computing (TCS).

The colors of the ST.

Inside the ST computer are sixteen hardware "registers", each of which contains

a code for producing a particular color on the monitor. The code is very

simple. Each color seen by the human eye is made up of various amounts of red,

green and blue light, which are produced by the ST's RGB (Red-Green-Blue)

monitor.

The ST is able to set each of the red, green and blue color components to one

of eight levels, numbered from 0 through 7. A level of 0 indicates that the

color is not present; a level of 7 indicates that the color is at its brightest

setting. The three colors,with eight values each, give the ST 8x8x8, or 512,

colors.

Normally, the ST can display only sixteen colors on the screen at one time

(there are only sixteen color registers in the computer). The user must pick

the sixteen colors he or she needs most and set the color registers

accordingly.

Fortunately for game designers and graphics aficionados like myself, the ST has

an interesting ability hidden under that gray exterior - an ability similar to

the display list interrupts (DLIs) of the 8-bit Atari computers. By using it,

the advanced programmer can change the ST color registers at any point on the

screen, giving the ST the power to display far more than 16 colors on the

screen at one time - even up to the system's limit of 512!

Before I go any further, I must explain that the STCP was developed by trial

and error, working with the hardware registers in the ST, and some of the

functions of the machine language subroutine cannot be fully documented without

complete information from Atari.

However, the macine language routine is documented enough for you to change it

to suit your needs. As more information about this capability is released by

Atari, ANALOG Computing will report it. I don't recommend using this procedure

in commercial programs, for some of the memory locations used are not yet

documented by Atari and may change in future revisions of the ST.

The full palette.

If you have a C compiler and assembler, type in, compile and link the programs

in Listings 1 and 2. If not, the ANALOG Computing TCS will have the compiled

program available to TCS subscribers.

This program must be executed in the 16-color, low-resolution mode, or it won't

work properly.

After loading the program, you'll see the 512 colors of the ST displayed on the

screen in 8 columns of 64 boxes. You can move the arrow on the screen, using

the mouse, and point to any color you like. That color's red, green and blue

settings will be shown in the lower right corner.

The program may be stopped at any time, and you can return to the GEM desktop

by pressing the SPACE BAR.

If you're not an advanced programmer - and don't want to be - enjoy the STCP.

If you are, read on for the details on how the ST can display its true colors.

It takes two.

As mentioned earlier, this program is made up of two parts: C language

initialization and control program and an assembly language section, which

contains special interrupt routines to enable the 512-color display.

The C control program, shown in Listing 1, gets everything started. Let's look

at the program and note the points of special interest.

+----------------------------------------------------------------------------+

| GEM Color Number    ST Pixel Value      Gem Color Number    ST Pixel Value |

+----------------------------------------------------------------------------+

|        0                   0                   8                   7       |

|        1                  15                   9                   8       |

|        2                   1                  10                   9       |

|        3                   2                  11                  10       |

|        4                   4                  12                  12       |

|        5                   6                  13                  14       |

|        6                   3                  14                  11       |

|        7                   5                  15                  13       |

+----------------------------------------------------------------------------+

Take a look at Figure 1 above. In the ST, the value of a pixel indicates which

color register is to be used for that color. To use color register 5, the

pixel's value should be 5.

Because GEM's color register numbering is not the same as the ST's pixel

numbers (GEM's color 5 gives a pixel value of 6, which uses color register 6),

it's necessary to translate the pixel values we want into their GEM

counterparts. This is done with the 8-member COFSET array. If we want the pixel

value to be 1, we tell GEM to use the first color index in the array, which is

2. If we want a pixel value of 3, we use the third index in the array, or 6.

This is a confusing way to set up the color registers, but the authors of GEM

must have had a good reason for doing it this way. Once this is done, though,

we don't have to worry about it anymore.

In order to tell the user the level of red, blue and green light present in

each color, we next set up the REDVAL, GRNVAL and BLUVAL text areas. Each of

these fields is two characters long, allowing for one digit and a terminating

character, the ASCII null (0). The null is the standard End-Of-Line (EOL)

character in ST text-handling routines.

A little further down, we SAVE the settings of the color palette so that they

may be restored before returning to the GEM desktop. I suggest that you always

do this, if you're going to change the colors in your programs - the user will

appreciate having the normal desktop colors restored. This operation uses the

BIOS SETCOLOR call, which, along with the other calls used in this program, is

documented in the "Hitchhiker's guide to the BIOS" manual from Atari. The

routines to call the BIOS are included in the machine language program in

Listing 2.

The next section of the program initializes the CIABLE array, which contains

the 512 2-byte integer values that define all the various colors available on

the ST. The STCP goes through three FOR loops to initialize the table, each

loop altering the red, green or blue component.

Note that the color values are added together with various multiplication

factors, to form a number such as $707 (7 red, 0 green, 7 blue), then added to

$F888. The resulting value is ready to be loaded into the ST's hardware color

register, and ranges from $F888 (black) to $FFFF (white). The CTABLE table is

externally defined, in the machine language portion of the program.

The next section of STCP draws 512 boxes on the screen, starting with color 1

on the left and ending with color 8 on the right. You'll see the columns of

boxes drawn on the screen, all the boxes in the same column being the same

color. When the machine language routine is called, the screen colors will be

changed every three scan lines, resulting in 512 colors on the screen instead

of 8!

After changing the colors to white for the arrow, and red, green and blue for

the color number readouts, the title message is printed and the arrowis shown

on the screen. We're now ready to start the colors!

To start the 512-color display, STCP issues a call to the machine languag GO.

This initializes and starts the 512-color display.

The next portion of the program reads the position of the arrow with the

VQ_MOUSE routine and gives a constant readout of the color register settings of

the color the arrow's pointing at. This is a handy way to figure out which

colors you want to use in a program - just point to the color you want, and the

red, green and blue digits in the lower right corner of the screen will tell

you what color settings to use.

The color register setting values are printed whenever the mouse moves to a new

color on-screen. When this happens, the digits representing the red, green and

blue values are placed in the REDVAL, GRNVAL and BLUVAL text messages, and are

printed to the screen in the corresponding color with the VST_COLOR and V_GTEXT

calls.

The keyboard is constantly monitored by the BCONSTAT and BCONIN functions - if

the SPACE BAR is pressed, the program exits.

Before exiting, STCP calls the machine language routine labeled STOP, shutting

off the routines we started earlier and restoring the system to normal

operation.

Finally, the color palette is returned to normal by using the SETPALLETE

(that's the way it's spelled in the "Hitchhiker's Guide...") routine to reset

the color registers to the values we saved when the program started.

The machine language routine.

Without adequate documentation from Atari, I can't guarantee the accuracy of

some of the assumptions I've made in the explanation of this machine language

routine. Many of the locations' functions listed are educated guesses, made by

removing the instructions and observing the results. This code is, however,

useful in exploring the use of horizontal blank interrupts on the ST, a

function similar to the DLI structure on the 8-bit Atari machines.

Unlike the 8-bit Atari computers, the 520ST doesn't have a "display list", the

special program that tells the display how to act. It does, however, have a

powerful ability to generate an interrupt which will perform a set of

instructions at various points on the screen. This interrupt can be programmed

to occur on every scan line, every two scan lines, every three scan lines, and

so forth. This is termed a horizontal blank (HBLANK) interrupt. We will also

use the vertical blank (VBLANK) interrupt ability to handle the control of the

HBLANK.

Listing 2 shows the machine language code used for the STCP. It consists of

four main routines: the initialization and startup code for the VBLANK and

HBLANK routines, the shutdown code for the VBLANK and HBLANK routines, the

HBLANK and VBLANK routines themselves, and the BIOS calls used by the C

program.

There are several equates defined at the start of the program. VBVEC is the

location of the vertical blank interrupt vector, a LONG (4-byte) location which

contains the address the system JMPs to on a vertical blank interrupt. HBVEC,

also a LONG value, tells the system where to JMP to on a horizontal blank

interrupt.

KEYVEC is apparently the vector used when the ST's intelligent keyboard

generates a message to the system (for a keyboard, mouse or joystick event).

The COLOR0-COLOR9 equates are the addresses of the color registers. COLOR0 is

at $FF8240, COLOR1 at $FF8242, and so on.

The first section of STCP, labeled GO, calls the SETUP routine in SUPERVISOR

MODE, a special configuration of the 68000 processor. In this mode, the program

may access any portion of memory, without restriction. Normally, a user's

program cannot access memory outside itself.

SETUP changes the background color (COLOR0) to black ($F888). It then saves the

registers we're going to alter in the HOLD locations and the SAVE locations.

After saving the important registers, STCP alters the registers we need. The

keyboard vector is altered to point to KEYVEC, the HBLANK vector (HBVEC) is

altered to point to our HBLANK routine, and the VBLANK vector (VBVEC) is

altered to point to VBLANK.

Several other registers are also changed, including $FFFA09, which shuts off

the keyclick to avoid interfering with the interrupt timing. The functions of

$FFFA07 and $FFFA13 are not yet understood, but the masking of $FFFA07 before

and after changing HBVEC seems to indicate that its low-order bit is an HBLANK

enable bit.

After setting up these registers, SETUP performs an RTS to return to the

calling program.

The next routine, STOP, once again calls a routine in supervisor mode. This

time, it calls RESTORE. RESTORE is responsible for replacing the values we

changed earlier, when we installed our own VBLANK and HBLANK routines. This is

an important operation, as it restores the original system configuration,

preventing system crashes when new programs are loaded into memory later. After

restoring the registers, RESTORE returns to the calling program with an RTS.

The next routine, MYKEY, is a short routine of one instruction added to

the@start of the keyboard interrupt handler. It simply changes the 68000 status

register to $2500, setting the priority of the keyboard handler to a lower

level, 5. This prevents keyboard or mouse events (key pressed or mouse moved)

from interrupting the HBLANK interrupt. Try removing this instruction, and

you'll see glitches on the color palette whenever you move the mouse or anytime

you press a key!

Note that the KEYSAVE location follows this routine - when the keyboard

interrupt occurs, it will set the status register to $2500, then use the $4EF9

as a JMP instruction to the address in KEYSAVE, which was the old keyboard

interrupt routine. We simply forced the system to execute our instruction

before performing its normal duties. This same technique is used on the VBLANK

routine.

The HBLANK routine is a simple routine, similar to a DLI routine on the 8-bit

Ataris. First, it saves the registers used (interrupts must do this), then gets

eight color values from the CTABLE [8 2-byte integers (WORDs) = 4 4-byte LONG

values, or 16 bytes], and places them into the color registers 1 through 8.

By moving the data in 4-byte LONG chunks, the interrupt needs fewer

instructions (4 LONG moves vs. 8 WORD moves). Afterward, it increments the

CTABLE pointer, CIX, by 16 for the next interrupt. It then restores the saved

registers and clears bit 0 in register $FFFA0F (presumably, to clear the

interrupt status), finally exiting the interrupt with the RTE (Return from

Exception) instruction.

The VBLANK routine, while also simple, contains a very important construct. The

first six lines are essential to setting up the HBLANK control. Remember how I

said we're executing an HBLANK interrupt every three scan lines? Well, this is

the code that determines that interval. The third and fourth lines tell the

system how often to generate the HBLANK interrupt, with the #3 value. Change

both of these to #4, and the interrupt will occur every four scan lines; change

them to #1, and the interrupt will occur every scan line. The surrounding code,

MOVE.B #0,$FFFA1B through MOVE.B #8,$FFFA1B, is necessary for proper

operation.

The next six lines of code grab the first eight color values from the CTABLE

array and place them inthe color registers. This sets the color for the first

line of boxes on the screen. Afterward, CIX is set to 16, ready for the HBLANK

routine. Remember, this code is executed every time the electron beam of the

monitor is at the top of the screen.

Finally, the VBLANK code adds 1 to color register 9, causing the title message

printed by the C program to cycle through various colors. The VBLANK code then

JMPs to the normal system VBLANK code, by using the $4FE9 (the JMP instruction)

followed by the saved VBLANK vector, VBSAVE.

The last portion of STCP is the code which allows the C program to call the

various BIOS routines via the 68000 TRAP statement.

After the program code is finished, the .DATA section defines the variables and

tables used by the program.

Have fun!

Although I'm not entirely sure of some of the functions of the registers used

by this program, I hope that it will encourage ST programmers to try

experimenting with the HBLANK feature of the 520ST. Like its counterpart on the

8-bit machines, this feature may be used to add color to many programs.

If you're not an advanced programmer, you can still use the ST Color Palette to

find color register settings and impress your friends with the colorful 520ST.

####################################################################