Documentation for

Gordo's MC68000 Tiny BASIC version 1.0

This  is  an adaptation of Li Chen Wang's  'Palo  Alto  Tiny

BASIC' for the Motorola MC68000 microprocessor.  It includes more

functions and program save and load. As distributed, it is set up

for  a Motorola MEX68KECB Educational Computer Board connected to

a  host  CP/M computer.  The source code should give  you  enough

details to allow you to install it on a different system.  If you

have any problems, you can write to me at:

Gordon Brandly

R.R. 2

Fort Sask., AB, Canada

T8L 2N8

This version for the Atari 520 ST translated by :

Kenneth L. Hurley

1230 W 7th #3

Eugene, OR  97402

(503) 343-7285

The Language

Numbers

In  this Tiny BASIC,  all numbers are integers and must be in the

range 2147483647 to -2147483648.

Variables

There are 26 variables denoted by the letters A through Z.  There

is  also a single array @(I).  The dimension of this array (i.e.,

the  range of value of the index I) is set automatically to  make

use  of all the memory space that is left unused by  the  program

(i.e.,  0 through SIZE/4, see SIZE function below). All variables

and array elements are 4 bytes long.

Functions

There are 4 functions:

ABS(X) gives the absolute value of X.

RND(X) gives a random number between 1 and X (inclusive).

SIZE gives the number of bytes left unused by the program.

PEEK(X) gives the value of the byte at memory location X.

Arithmetic and Compare Operators

/    Divide. (Note that since we have integers only, 2/3=0)

*    Multiply.

-    Subtract.

+    Add.

>    Greater than. (comparison)

<    Less than. (comparison)

=    Equal to.  (comparison)   Note that to certain versions

of BASIC "LET A=B=0" means "set both A and B to 0".  To

this  version  of Tiny BASIC,  it means "set A  to  the

result of comparing B with 0".

<>   Not equal to. (comparison)

>=   Greater than or equal to. (comparison)

<=   Less than or equal to. (comparison)

+,  -,  *,  and / operations result in values between -2147483647

and 2147483647.  (-2147483648 is also allowed in some cases.) All

compare operators result in a 1 if true and a 0 if not true.

Expressions

Expressions  are formed with numbers,  variables,  and  functions

with arithmetic and compare operators between them. + and - signs

can also be used at the beginning of an expression.  The value of

an expression is evaluated from left to right,  except that * and

/  are  always done first,  and then + and - ,  and then  compare

operators.  Parentheses  can also be used to alter the  order  of

evaluation.  Note  that  compare  operators can be  used  in  any

expression. For example:

10 LET A=(X>Y)*123+(X=Y)*456+(X<Y)*789

20 IF (U=1)*(V<2)+(U>V)*(U<99)*(V>3) PRINT "Yes"

30 LET R=RND(100), A=(R>3)+(R>15)+(R>56)+(R>98)

In line 10,  A will be set to 123 if X>Y,  to 456 if X=Y,  and to

789 if X<Y. In line 20, the "*" operator acts like a logical AND,

and the "+" operator acts like a logical OR.  In line 30,  A will

be a random number between 0 and 4 with a prescribed  probability

distribution of: 3% of being 0, 15-3=12% of being 1, 56-15=41% of

being 2, 98-56=42% of being 3, and 100-98=2% of being 4.

Program Lines

A  Tiny BASIC line consists of a line number between 1 and  65534

followed  by one or more commands.  Commands in the same line are

separated by a colon ":".

"GOTO",  "STOP",  and  "RETURN"  commands must be  the  last

command on any given line.

Program

A Tiny BASIC program consists of one or more lines. When a direct

command  "RUN"  is issued,  the line with the  lowest  number  is

executed  first,  then the one with the next lowest line  number,

etc.  However, the "GOTO", "GOSUB", "STOP", and "RETURN" commands

can  alter  this normal sequence.  Within a  line,  execution  of

commands  is from left to right.  The "IF" command can cause  the

remaining commands on the same line to be skipped over.

Commands

Tiny BASIC commands are listed below with examples. Remember that

multiple commands can be put on one line if colons separate them.

In  order to store the line,  you must also have a line number at

the beginning of the line.  (The line number and multiple-command

lines aren't shown in the examples.

REM or REMARK Command

REM anything goes

This line will be ignored by Tiny BASIC.

LET Command

LET A=234-5*6, A=A/2, X=A-100, @(X+9)=A-1

Will  set  the variable A to the value of the expression  234-5*6

(i.e.  204),  set  the  variable A (again) to the  value  of  the

expression A/2 (i.e. 102), set the variable X to the value of the

expression A-100 (i.e. 2), and then set the variable @(11) to 101

(where 11 is the value of the expression X+9 and 101 is the value

of the expression A-1).

LET U=A<>B, V=(A>B)*X+(A<B)*Y

Will  set the variable U to either 1 or 0 depending on whether  A

is  not  equal  to or is equal to B;  and set the variable  V  to

either  X,  Y or 0 depending on whether A is greater  than,  less

than, or equal to B.

Print Command

PRINT

Will  cause  a carriage-return (CR) and a line-feed (LF)  on  the

output device.

PRINT A*3+1, "abc 123 !@#", ' cba '

Will  print the value of the expression  A*3+1  (i.e.  307),  the

string  of  characters "abc 123 !@#" and the string" cba  ",  and

then  a CR-LF.  Note that either single or double quotes  can  be

used to quote strings, but pairs must be matched.

PRINT A*3+1, "abc 123 !@#", ' cba ',

Will produce the same output as before,  except that there is  no

CR-LF  after the last item printed.  This enables the program  to

continue printing on the same line with another "PRINT".

PRINT A, B, #3, C, D, E, #10, F, G

Will  print the values of A and B in 11 spaces,  the values of C,

D,  and E in 3 spaces, and the values of F and G in 10 spaces. If

there  aren't  enough  spaces specified for a given value  to  be

printed, the value will be printed in full anyway.

PRINT 'abc',_,'xxx'

Will  print the string "abc",  a CR without a LF,  and  then  the

string "xxx" (over the "abc") followed by a CR-LF.

INPUT Command

INPUT A, B

When  this  command is executed,  Tiny BASIC will print "A:"  and

wait to read in an expression from the input device. The variable

A  will  be set to the value of this  expression,  then  "B:"  is

printed and variable B is set to the value of the next expression

read  from the input device.  Note that whole expressions as well

as numbers can be entered.

INPUT 'What is the weight'A, "and size"B

This is the same as the command above,  except the prompt "A:" is

replaced by "What is the weight:" and the prompt "B:" is replaced

with  "and size:".  Again,  both single and double quotes can  be

used as long as they are matched.

INPUT A, 'string',_, "another string", B

The strings and the "_" have the same effect as in "PRINT".

POKE Command

POKE 4000+X,Y

This  command puts the value produced by expression "Y" into  the

byte memory location specified by the expression "4000+X".

CALL Command

CALL X

This  command  will  call a machine language  subroutine  at  the

address  specified  by  the expression  "X".  All  of  the  CPU's

registers   (except  the  stack  pointer)  can  be  used  in  the

subroutine.

IF Command

IF A<B LET X=3: PRINT 'this string'

This will test the value of the expression A<B.  If it isn't zero

(i.e.  if it is true), the rest of the commands on this line will

be executed.  If the value of the expression is zero (i.e.  if it

is  not  true),  the rest of this line will be skipped  over  and

execution continues on the next line.  Note that the word  "THEN"

is not used.

GOTO Command

GOTO 120

Will  cause execution to jump to line 120.  Note that the  "GOTO"

command cannot be followed by a colon and other commands. It must

be ended with a CR.

GOTO A*10+B

Will  cause  the execution to jump to a different line number  as

computed from the value of the expression.

GOSUB and RETURN Commands

GOSUB 120

Will cause execution to jump to line 120.

GOSUB A*10+B

Will cause execution to jump to different lines as computed  from

the value of the expression A*10+B.

RETURN

A  RETURN command must be the last command on a line and must  be

followed by a CR.  When a RETURN command is encountered,  it will

cause  execution  to jump back to the command following the  most

recent GOSUB command.

GOSUB's can be nested with a depth limited only by the stack

space.

FOR and NEXT Commands

FOR X=A+1 TO 3*B STEP C-1

The  variable X is set to the value of the  expression  A+1.  The

values  of  the expressions (not the expressions themselves)  3*B

and  C-1 are remembered.  The name of the variable  X,  the  line

number  and the position of this command within the line are also

remembered.  Execution then continues the normal way until a NEXT

command is encountered.

The step can be positive,  negative or even zero.  The  word

STEP  and  the  expression  following it can be  omitted  if  the

desired step is +1.

NEXT X

The  name  of  the variable X is checked with that  of  the  most

recent  FOR  command.  If they do not agree,  then  that  FOR  is

terminated and the next recent FOR is checked,  etc. When a match

is found, this variable will be set to its current value plus the

value  of  the  step expression saved by  the  FOR  command.  The

updated  value  is  then  compared  with  the  value  of  the  TO

expression  also  saved by the FOR command.  If this  within  the

limit,  execution will jump back to the command following the FOR

command.  If  this  is  outside the  limit,  execution  continues

following the NEXT command itself.

FOR's  can also be nested with a depth limited only  by  the

stack space.  If a new FOR command with the same control variable

as that of an old FOR command is encountered, the old FOR will be

terminated automatically.

STOP Command

STOP

This  command  stops  the execution of the  program  and  returns

control  to direct commands from the console.  It can appear many

times  in  a program but must be the last command  on  any  given

line, i.e. it cannot be followed by a colon and other commands.

BYE Command

Will  return  you  to the resident monitor program  or  operating

system.  (Tutor in the case of the ECB.)

Direct Commands

As defined earlier,  a line consists of a line number followed by

commands.  If  the  line number is missing,  or if it is  0,  the

commands  will be executed after you have typed the CR.  All  the

commands  described above can be used as direct  commands.  There

are  five  more commands that can be used as direct commands  but

not as part of a program line:

RUN

Will  start  to  execute  the  program  starting  at  the  lowest

line number.

LIST

Will print out all the lines in numerical order.

LIST 120

Will  print out all the lines in numerical order starting at line

120.

NEW

Will delete the entire program.

SAVE

Will save the present program on the storage device you  provide.

Details  on installing this device are given in the source  code.

As  set  up for the MEX68KECB Educational  Computer  Board,  this

command  will  send  the program out to the host computer  in  an

easily-stored text form.  It isn't, however, pure program text as

the line numbers are stored in hexadecimal.

LOAD

Will  delete  the program in memory and load in a file from  your

storage device.

Stopping Program Execution

The  execution  of the program or listing of the program  can  be

stopped   by   pressing  the  control-C  key  on   the   console.

Additionally,  a listing can be paused by pressing control-S, and

then pressing any key to continue.

Abbreviations and Blanks

You  may use blanks freely within a program except that  numbers,

command  key  words,  and  function names  cannot  have  embedded

blanks.

You  may  truncate all command key words and function  names

and follow each by a period.   "P.",  "PR.",  "PRI.", and "PRIN."

all stand for "PRINT".  The word LET in LET commands may also  be

omitted.  The shortest abbreviations for all the key words are as

follows:

A.=ABS    C.=CALL   F.=FOR    GOS.=GOSUB     G.=GOTO

IF=IF     I.=INPUT  L.=LIST   LO.=LOAD       N.=NEW

N.=NEXT   P.=PEEK   PO.=POKE  P.=PRINT       REM=REMARK

R.=RETURN R.=RND    R.=RUN    S.=SAVE        S.=SIZE

S.=STEP   S.=STOP   TO=TO

no key word = LET

Error Reports

There  are  only three error conditions in Tiny BASIC.  The  line

with  the error is printed out with a question mark  inserted  at

the point where the error is detected.

(1) "What?" means it doesn't understand you.  For example:

What?

260 LET A=B+3, C=(3+4?. X=4

(2) "How?" means it understands you but doesn't know how to do it.

How?

210 P?TINT "This"   <- where PRINT is misspelled

How?

310 LET A=B*C?+2    <- where B*C is larger than 2147483647

How?

380 GOTO 412?       <- where 412 does not exist

(3)  "Sorry." means it understands you and knows how to do it but

there isn't enough memory to accomplish the task.

Error Corrections

If you notice an error in your entry before you press the CR, you

can delete the last character with the backspace (control-H)  key

or delete the entire line with control-X.  To correct a line, you

can  retype the line number and the correct commands.  Tiny BASIC

will  replace the old line with the new one.  To delete  a  line,

type  the  line  number  and  a  CR  only.  You  can  verify  the

corrections to line 'nnnn' by typing "LIST nnnn" and pressing the

control-C key while the line is being printed.