ARCHIVE.DOC

An interest has been expressed recently in giving Citadel the

ability to automatically maintain, in some way, a record of

conversations that are held.  This has been motivated by the

CitaBook project, story-telling, and other subjects that have, for

one reason or another, given the system operator reason to pause,

think, and wish to save those messages in a far more permanent form

than as a Citadel message, which will inevitably scroll.

The first attempt at such an ability was the EXTRACT utility.

Crappily written, an awkward user interface, and being an

independent utility all contributed to it being spat upon by all who

used it (including the author).

The second attempt, which is itself recent as of this date, was

essentially the integration of EXTRACT into CTDL.EXE.  This feature

was restricted to aides at the sysConsole, and was accessed in a way

similar to the manner in which messages are deleted.  The aide would

hit <P>ause during the printout of the message to be extracted.

Then the aide would hit J, for Journal, to restart the message (to

Delete a message, the aide would hit D).  At the end of the message,

Citadel would ask for the file to store the message under. This is

(I think) a very useful ability, and seems to work well.

Still, it boils down to a lot of manual labor; it's only

convenient for single messages.  What if the sysop wants to record

an entire room on disk, on an ongoing basis?

Therefore, Citadel now has the ability to maintain just such a

textfile on disk. Called (for the time being) the Archive feature,

it allows the sysop to designate virtually any room to be an Archive

room. Such a room then will have a filename associated with it.

Within the file of that name, each and every message that is entered

into that room will also be appended to a archive file.

PROCEDURE:

The procedure to designate a room as Archive is to simply edit

the room (note that this automatically excludes Aide>, Mail>, and

the baseRoom as Archive rooms).  One of the questions that you will

encounter will concern Archive status of the room. If you answer yes,

the system will do one of two things:

A) If the room was not an Archive room before, it'll ask for a

filename to associate with the room.

B) If it was already an Archive room, it'll just assume that the

filename should remain the same, and not bother you further about

Archive stuff.

If it does ask for filename, the filename that you should type

in can be located anywhere in the system that you desire.  Once you

have typed it in, Citadel will attempt to archive all the messages

that are currently in the room (message numbers will be spewed out

at this point to reassure you that there is* method in Citadel's

madness).

An Archive room puts each new message into the Archive file when

it is initially entered into the room (note that Aide actions, such

as deleting the message later, does not affect the Archive file).

CONSIDERATIONS:

This feature is recommended only for high-performance systems,

particularly those with fairly fast hard disks, since each message

is, in effect, saved twice.  Furthermore, since active rooms will

result in very fast file growth, close tabs should be kept on disk

free space by the sysop.

NOTES:

This is all Torkildson's fault.

The filenames are stored in the new .SYS file CTDLARCH.SYS file.

This file is unique in the respect that it is a human readable text

file. Each Archive room on the system will have an entry in this file,

in the following format:

<room#><space><full-filespec>

Thus, if the sysop wants to modify what file to store the

Archive data in without going through the hassle of editing the room

twice (see above), s/he can whip out their handy-dandy text editor,

and fix it right then and there.