FROM MS-DOS BBS TRIANGLE PARK, N.C.

Line Noise and the Problems it Can Cause with File Transfers

Many people have left messages on my bulletin board asking me why there

are so many 'garbage' characters on their screens and why file transfers

are riddled with errors. These garbage characters are really line noise

and can be introduced in many different places. Pure noise is a decimal

255 (FF inhex), but most line noise is not 'pure'. It usually comes in

as something less than 255, like maybe a 251 (a character that looks

like this '{'). Ever see that one before? Yup, so have I!

One of the more common and familiar introduction points of line noise is

in the telephone company's system and even here there are several ways

noise is introduced. A signal is routed through multiple stations before

it eventually makes it to the other end and some of these stations aren't

exactly new. Older areas may have older, less sophisticated equipment

that is more apt to be affected by ambient noise. This is one reason some

people continue to have noise problems even after hanging up and calling

back multiple times.  Also, a given physical connection at one of these

junctions may not be up to snuff. If your particular bout of line noise

is solved by hanging up and calling back, then it's probable that you

were previously connected through an intermittent or 'dirty' connection.

Some of these trunk lines (large, multi- area that has alot of ambient

RFI (RadioFrequency Interference) present although this is not usually

the case.

It is possible that the problem is being caused at this end, but not if

the problem goes away when you call back and the line is clean -or- if

you are one of a very few users experiencing noise problems. You may say

that you are not having problems with other boards.....in which case the

problem is more than likely the route that your call takes to get here.

You may be going over micro-wave or through buried cable which for some

reason are sub-standard. No matter how many times you call, you will

probably be routed over the same path. Micro-wave problems are sometimes

the hardest to track down because they can cause intermittent problems.

Some interference only occurs during certain times of the day or week.

Another common noise introduction point is in your home. Most residential

homes have televisions, radios, microwave ovens, VCR's, and if you are

reading this, a micro-computer. All these devices radiate radio waves

that can (and often do) get into the phone lines and cause noise.

Electric motors and mechanical dimmer controls can introduce noise into

the electrical wiring in your house and cause problems. If your line

noise problem does not go away after repeated hanging up and calling back,

then you may be suffering from one of these household problems. If you

are suffering from this problem, you can take steps to eliminate it.

First of all, turn off EVERTHING except the fridge (If it IS the fridge,

then you're SOL. Can't live life with your ice box unplugged) and see if

the noise persists. If it goes away, then start turning things back on,

checking the computer each time until you see the noise start up again.

It may be that a single device is not bugging you but several devices

plotting together to annoy you. This elimination tournament may take

awhile.

Another area to check is your wiring at the computer. Use noise

supressors on your power connections to both the PC and the modem (if

external). Use a shielded RS-232 cable to connect your modem to the PC.

Ribbon cables (especailly long runs of it) are great antennas and will

cause problems. Re-route the RS-232 cable so it does not run next to the

PC power supply or any other transformer.

And now a little discussion about the modem itself. First of all, I'd

like to clerify a commonly misused term - BAUD. The term "Baud" is

actually a man's name - J.M.E. Baudot (Pronounced: Baw-doe) a French

Telegraphy expert. 1,200 and 2,400 Baud is NOT the same as 1,200 and

2,400 BPS (Bits Per Second).  The usage of "Baud" to describe line speed

in terms of data through-put is incorrect. 1,200 and 2,400 BPS modems

both operate at 600 Baud. Basically, without getting to technical, a

Baud is a "blip" of information. 1,200 BPS modems use four states per

blip (or Baud) and 2,400 BPS modems use sixteen states per blip. If you

want more information on what Baud and BPS mean and a full explanation

of how data is actually represented and transferred by the modem, please

refer to PC Magazine Volume 6, Number 9 (May 12, 1987).

Modems operating at 2,400 BPS are much more intolerant of line noise than

are modems operating at 1,200 BPS. Conversely, modems capable of 2,400 BPS

operate better at 1,200 BPS than do 1,200 BPS only modems. If you are

being hopelessly attacked by noise at 2,400 BPS, trying calling back at

1,200 BPS.  It's very possible that the noise will be greatly reduced

or disappear altogether. I know, you didn't buy a 2,400 BPS modem just

to retard it to 1,200 BPS. The brand of the modem plays a part in the

immunity to line noise.  Some modems can digest more noise (lower

signal-to-noise radio) than others.  PC Magazine (same issue mentioned

above) ran a test on 87 different modems.  You might check the results

to see how your modem ranks. Most 2,400 BPS modems operating at 1,200 BPS

have approximately -8 to -10 db error threshold while the same modem

has about -16 to -20 db threshold operating at 2,400 BPS.  For this

reason, line quality is much more critical at 2,400 BPS operation.

Additionally, a friend of mine who runs a bulletin board from their

office has been plagued with line noise problems at 2,400 BPS but very

little noise at 1,200 BPS. The culprit is the office's centralized

telephone system. Many office buildings have a given number of trunks

that actually enter the building while there may be many, many more

extension within the building. These types of telephone systems have

their own controllers and line assignment devices and are frequently

not as high in quality as a hard-wired MaBell (or GTE) line. The

acceptable signal-to-noise ratio in some of these inter-office phone

controllers are lower than necessary for reliable 2,400 BPS operation

but not too low for 1,200 BPS.

If you get transmission errors while downloading or uploading a file,

don't fret it. The Xmodem (or whatever protocol) incorporates an error

checking/correction mechanism that automatically detects and corrects any

errors that may occur during transmission. The very fact that Xmodem

reported the error in the first place means that he caught it and

corrected it. The only errors you have to worry about are the ones that

Xmodem does NOT report  Any reported error has already be corrected.

Xmodem, especially the CRC flavored one, is a very reliable file transfer

protocol. Even if you got 100 errors during transmission, chances are

still pretty slim that the file got corrupted. Occasionally, a file

will be corrupted after transfer, but many times this may be due to a

bad ARCing of the file or perhaps a disk error that may have occured

sometime during the files' past.

I hope this text helped explain some facts about modems, line noise, and

file transfers. If you have other, more specific questions, concerning modems

or communications in general, leave a C)omment to the SYSOP. I'll try to

answer them.###################################################################################################