EmuXL version 0.3.x

-------------------

This is only a preview beta version, the emulator is still under development.

Not many features have been implemented so far. For now, you can play Atari

BASIC, and two graphic modes (GRAPHICS 0, i.e. 40x24 characters monochrome, and

GRAPHICS 8/24, 320x192 pixels monochrome). On a stock Falcon the speed isn't

great, about 30-35% of the original using default settings. Any hardware

expansions may take effect in more speed, esp. CPU accelerators (quite obvious)

and FastRAM. Graphic cards are not supported.

The emulator is designed and proved to work with plain TOS as well as with

MultiTOS or MagiC, though that last was reported to feature some doubtful

effects, which are gone now without any special reason. The emulator is being

developed on an ordinary Atari Falcon030 with 14 MB RAM running MiNT (FreeMiNT

kernel version 1.14.5) and N.AES 1.1.0.

The emulator is designed to work on any 68k processor later than 68010. It is

also proved to work on Afterburner040 (68040/40 MHz).

WARNING TOS USERS!

The emulator does several GEMDOS calls at startup in order to allocate

something like 15 blocks of memory. As the TOS 4.04 allows you to allocate only

32 blocks of memory in total (without releasing anything), please don't be

surprised if the system hangs after loading Emu! If such a problem occurs for

you, please reduce the number of AUTO programs and accessories, or use MultiTOS

instead of the TOS (make sure you have MiNT kernel version 1.12.6 or newer).

Setting up the Emu

==================

Just unpack the archive anywhere you want to then run the file EMU.PRG from the

Desktop. Now you can set some options in dialog boxes. These are as follows:

CPU & OS configuration - provides settings which may alter the manner the 6502

CPU is emulated, operating system selections and so on.

Video configuration - provides settings which may alter the manner the Antic

video chip is emulated.

To run the emulation, just select "Cold start" or "Warm start" from the menu.

The screen should get black then while the XL OS is being restarted. After a

while you should see nice blue screen with a white cursor in the upper right

corner. Few seconds later a "READY" should appear. It is a prompt of Atari

BASIC, the built in BASIC interpreter.

If you saw a "BOOT ERROR" message instead of "READY", that means that the XL OS

attempted to boot off from a (virtual) disk and failed. Actually it will always

occur, because no virtual disk images are provided with the current version of

the emulator. To get rid of boot errors you have to setup "GEMDOS filesystem

access" properly.

If you select "Restore & restart", the emulator will attempt to load a

previously saved RAM image. If it succeeded (there's such image file in the

distribution, containing a very simple program written in BASIC), you may

return to anything you did last time.

Key functions

=============

For now, only US keyboard layout is supported, so what you get may differ from

that you pressed if you're using a German, British or any other keyboard

layout.

Besides of the main block, the emulator supports numpad and cursor keys. Some

keys have special meaning, corresponding to special keys found on XL/XE

keyboard.

F5 key aborts the emulation.

Falcon keyboard		Function

---------------		--------

F6			HELP key

F7			START key

F8			SELECT key

F9			OPTION key

F10			RESET key

Alternate		Inverse video

Undo			Break

Help			HELP key

ClrHome			Clear (Shift <)

Shift/ClrHome		Cursor Home (1200XL keyboards only)

Left mouse button	Joystick #0 fire

Right mouse button	Joystick #1 fire

Special key combinations:

Control/1		Display output start/stop

Control/2		Warning buzzer

Control/3		Send end-of-file to the editor

Control/Tab		Clear tabulation

Shift/Tab		Set tabulation

Control/CapsLock	Set keyboard to semigraphic

Shift/CapsLock		Set keyboard to uppercase

Shift/Delete		Delete line at cursor position

Shift/Insert		Insert line at cursor position

Control/Backspace	Delete character right from cursor position

Control/Insert		Insert character at cursor position

Testing emulation speed

=======================

To test emulation speed on your system, please enable the Falcon filesystem

access (click the "Enable driver" button in the appropriate dialog box), reboot

the XL OS, then dispose:

LOAD "D:CIRCLE.BAS"

at the Atari BASIC prompt. When the desired file has been loaded with no errors

to the memory, type:

RUN

then press Return key. The program will switch to a bitmap mode and draw a

circle using trigonometric functions. After it finishes, it displays the number

of seconds it ran. The circle is drawn about 53.44 sec. on the original 8-bit

Atari.

On a stock Falcon030 the speed isn't great and may depend on a job the emulator

is doing. I've supplied some BASIC programs to test it, these are:

LOOP.BAS - runs a simple FOR/NEXT loop. The speed is about 34%

CIRCLE.BAS - draws a circle in GR.8 using SIN/COS. The speed is about 40%

SMPLTXT2.BAS - runs a 24-bit loop. Speed: 57%

So, programs running in bitmap modes are generally faster than those running in

text mode (quite obvious). Short loops (those probably fit in the cache) run

faster than anything else, etc. Testing the speed please remember that the

RTCLOCK these programs are using is VBL driven. So if you have accelerated

Falcon, you probably have higher VBL rate than 50Hz...

To get a directory listing from within the Atari BASIC, please dispose:

RUN"D:DIR.BAS"

Saving RAM image

================

To save the current state of emulator's memory, press F5 (abort emulation).

Once you do it, an alterbox should appear displaying some debug information

(mainly the current CPU state). If you click "Dump RAM" button this time, the

emulator's memory contents will be written to the disk. Of course, the previous

RAM image, if any, will be destroyed.

CPU config dialog

=================

Most of these options aren't yet alterable, they'll be subsequently implemented

in the future.

Instruction set - 6502 or 65c02 instruction set. For now only that last is

supported.

Illegal instructions - "no operation" means all codes unassigned to 6502

instructions are treated as NOPs.

Hardware type - will select between 400/800 and 800XL/130XE hardware type. At

this stage only a 400/800 computer with 40k RAM is emulated.

Operating system - will allow to select the OS image to load. XL OS is loaded

by default.

Floating point - the way the floating point calculations will be done:

- emulate 6502, calculations are done by the 6502 routines supported by the XL

OS.

- use MC 68030, the emulator will use 68030 as a sort of arithmetical

coprocessor. Calculations are done outside the XL OS by internal routines

written in 68030 assembly language. It speedups all software using FP

package noticeably, especially all BASIC programs and the interpreter itself.

Some complex calculations, e.g. the raise to power function, may be even

nine times faster, than on an original XL. Notice however, that when this

"coprocessor" is running, the 6502 remains halted and no timers are updated;

so, if you measure the time something is running, the result may be false.

GEMDOS filesystem access - this is provided for a special CIO device allowing

to access the real TOS filesystem from the emulator level.

Video configuration dialog

==========================

Video display - this option is provided to support external graphic cards. Only

internal video hardware is supported for now.

Screen refresh rate - allows to set, how often the screen display will be

refreshed. The bigger it is, the graphics is smoother, but the CPU

emulator runs slower. Normal Falcon may hardly run at 50 Hz screen

refresh. These settings don't affect the rate the XL OS VBL routines

are executed with. That last is (or should be) equal to the current

Falcon framerate.

Screen refresh method - the way the screen display will be refreshed:

- ST RAM direct write, data are written directly to the screen memory.

- use FastRAM buffer, data are written to the Fast RAM (if present), then the

Emu blits them to the screen memory. If the Fast RAM is fast enough, it

should speedup the emulation.

Vertical blank interrupts - the way the VBL service routine is executed:

- follow NMI vector, NMIVEC at $FFFA is used.

- use VBLKI vector, immediate VBL vector at $0222 is used. It may speedup the

emulation a bit by reducing the VBL interrupt overhead.

Display List Interrupt - the way the DLI service routine is executed:

- follow NMI vector, as above.

- use DLI vector, Display List Interrupt vector at $0200 is used. This reduces

the DLI overhead.

Video mode:

- use 31 kHz mode for SVGA monitors

- use 15 kHz mode for low resolution, TV or multisync monitors. That last I

mean a true multisync, i.e. a monitor able to work at 15.625 kHz for

horizontal frequency, not modern crap called "multisync" while it can do

only frequencies above 30 kHz. An example of a true multisync monitor is the

NEC Multisync 3D or Microvitec 4235.

The emulator uses 384x240/16 display mode either on SVGA or on RGB. The SVGA

framerate is tuned down to 50 Hz.

Debug info

==========

After the emulator is aborted, an alertbox will appear providing some debug

information. Essentially, it's the current CPU (6502) state. The following

information is displayed:

A - accumulator value

X - index register X value

Y - index register Y value

S - stack pointer value. Default is $FF (empty stack)

P - processor status register

PC - program counter

CPU time - the time spent emulating 6502 in seconds.

Most these values are quite obvious. The processor status register (P) contains

flags indicating a result of last executed instruction. From the left, bits are

as follows:

N - negative, last operation set the highest (7) bit of a register.

V - overflow, last operation changed the 6th bit of a register

* - this bit is unused, should be always set.

B - BRK instruction was executed

D - decimal mode for integer arithmetics

I - interrupts (IRQ line) are disabled.

Z - zero, last operation zeroed a register

C - carry, last addition produced a result bigger than 255 in binary mode or

bigger than 99 in decimal mode. For subtraction, if the result was less

than a zero, this bit is cleared (it's borrowed).

The first difference between the 6502 and what you may know from Motorola

processors is, that the P register in the 6502 indicates the *processor*

status. That means, that this register will be altered by any calculations done

internally by the CPU as well as by any data loaded to a CPU register from the

memory, but it is NOT altered while writing data from a register to the memory.

For example, if you load a zero to the accumulator, the Z bit will be set. If

you load a $01 to the X register, the Z bit will be cleared. But if you store

the (previously zeroed) accumulator to the memory, the Z bit remains cleared.

Just because this operation doesn't alter the internal processor state.

The next difference is the C (carry) bit function. First of all, it's functions

for addition and for subtraction are different. Addition sets the C if the last

result was bigger than it could fit on 8 bits, while subtraction *clears* this

bit if the result was less than a zero (also comparisons clear the C that time).

The fourth big difference between 6502 and 68k series is that all operations

affect only flags referencing to results they can produce. That means, for

example, that logical operations like AND, OR, EOR affect NZ only, because such

an instruction simply can't cause an overflow, no result can be bigger than 255

or less than a zero.

The last big difference is that all 6500 processors are little endian, while

all 680x0 chips are big endian. This refers to the format of integer

representation. All little endians (6500, Z-80, 80x86) place the low

significant byte of an integer at the first position, while all big endian

processors place there the most significant byte.

XLD file format

===============

The XLD files keep disk image data for the emulator. They consist of a header

and body. The header is 16 bytes long:

Offset	Value	Meaning

------	-----	-------

0	EXLD	magic value indicating it's a EmuXL disk image file

4	byte	number of tracks

5	byte	nothing

6	word	number of sectors per track

8	byte	number of disk sides decreased by 1 (1 for a double sided disk)

9	byte	nothing

10	word	number of bytes per sector

12	byte	nothing

13	byte	nothing

14	byte	nothing

15	byte	nothing

All words above are in the big endian format. The header is followed by

subsequent disk sectors, starting from sector number 1.

GEMDOS filesystem access (formerly called "Falcon File System", FFS)

====================================================================

Besides of using virtual disks kept by the emulator as XLD files, the Emu

allows you to have files stored directly on real GEMDOS filesystems. Notice

however, that for now it's not possible to use XLD images and FFS access AT THE

SAME TIME. It will possible in future versions of Emu. For now FFS, if enabled,

prevents the XL OS from booting from a virtual diskette.

The FFS access is configured using the dialog box entitled "Filesystem access

configuration". To enable this feature, please select a device identifier for

the XL OS (the best is D: or F:), then assign particular units (D1:, D2:, D3:

etc or F1:, F2:, F3: etc depending on the selected ID) to appropriate Falcon

filesystems (disks from A: to P:). Particularly, an unit may be unassigned

("nothing") or may be assigned to the "default" GEMDOS filesystem. The default

filesystem is "mounted" on the subdirectory XLFS, that is found inside the

Emu's application folder.

If you select "Read only" for a particular filesystem, the emulator (exactly:

the XL OS) won't be able to create a file and write data there, so status 144

(write protection) will be returned.

Now select the button "Enable driver", then reboot the XL OS ("XL OS cold

start" from the menu).

8-bit Atari pathnames slightly differ from what you may know from GEMDOS. They

always start from a device identifier, that may be followed by a particular

unit number and a colon. Next part is the pathname itself. For example:

"D:CIRCLE.BAS" means a file called CIRCLE.BAS stored on a device "D"

(presumably and traditionally its a disk filesystem). No unit number usually

means unit number 1, but may also mean a default unit. Unit number may be

specified by a number from 1 to 9 or a character from A to I (also lowercase

letters are recognized properly).

Subdirectories are specified with the character ">" instead of backslash,

though *in the Emu* backslashes should work too. So, "D:>EMU>CIRCLE.BAS"

specifies the file as above in a directory EMU existing in the root directory

of a filesystem.

Notice, that the FFS, although it's a very nice feature, is much slower than

virtual disks, where data are transferred in portions 128 or 256 bytes each.

The FFS transfer is byte-by-byte and in fact it's even slower than on a real 8-

bit machine using a real disk drive.

WARNING! GEMDOS filesystem device uses some feature of the XL OS, that is NOT

supported by the older system found in Atari 400/800 computers. So, if you

replace the OS.SYS with an older version of the system, this device WON'T WORK.

Enjoy,

Konrad M.Kokoszkiewicz

draco@mi.com.pl