Memo:	Hard Disk Partitioning

Date:	29 November 1988

From:	Minna Lai

To:	Henry Plummer

cc:	L. Tramiel, A. Pratt, R. Good, A. Char

The following is a proposal for the layout of partition information on a hard disk connected to a TT.  The layout is identical to the current one used on the SH204, MEGAFILE 30 and MEGAFILE 60 hard disks.

The first sector (physical sector #0) on a hard disk looks like:

Offset

+-------------------------------------------------------+$0

|		|

|	boot code (if any)	|

|		|

+-------------------------------------------------------+$1c2

|		|

|	hd_siz	|

|		|

+-------------------------------------------------------+$1c6

|		|

|	partinfo_0	|

|		|

+-------------------------------------------------------+$1d2

|		|

|	partinfo_1	|

|		|

+-------------------------------------------------------+$1de

|		|

|	partinfo_2	|

|		|

+-------------------------------------------------------+$1ea

|		|

|	partinfo_3	|

|		|

+-------------------------------------------------------+$1f6

|		|

|	bsl_st	|

|		|

+-------------------------------------------------------+$1fa

|		|

|	bsl_cnt	|

|		|

+-------------------------------------------------------+$1fe

|		|

|	(reserved)	|

|		|

+-------------------------------------------------------+$200

hd_siz (4 bytes)

- the total size of the disk in number of sectors.

bsl_st (4 bytes)

- the starting sector number of the bad sector list.

(Typically will be physical sector #1)

bsl_cnt (4 bytes)

- the size of the bad sector list in number of sectors.

partinfo_n (n = 0, 1, 2, 3) (12 bytes each)

- a 12-byte structure to describe a partition

+---------------------------------------------------------+   (+0)

|	p_flg	|

+---------------------------------------------------------+   (+1)

|	p_id	|

+---------------------------------------------------------+   (+4)

|	p_st	|

+---------------------------------------------------------+   (+8)

|	p_siz	|

+---------------------------------------------------------+   (+12)

p_flg (1 byte)

- bit 0 set indicates the partition exists

- bit 7 set indicates partition is bootable

p_id (3 bytes)

- a three-byte field to identify the partition

(e.g.  "GEM" for GEMDOS partitions)

p_st (4 bytes)

- physical sector # of the beginning of the partition.

p_siz (4 bytes)

- size of the partition in number of sectors