What is the Bad Sector List?

Terms

Several terms are used in this document which are best described here:

BSL:

Bad-Sector List.

Vendor bad sectors:

Bad sectors reported by the vendor of the hard disk device, and those not recoverable by reformatting the device (more detail later).  Contrast with user bad sectors.

User bad sectors:

Those bad sectors found by a "Markbad" type utility run by the user.  These are suspect: it is possible that these will be recoverable by reformatting.

Media BSL:

The bad-sector list placed at the start of the media.  This list contains information on both user and vendor bad sectors, in such a way that they can be distinguished.  During formatting, the user part of this list is ignored, and the vendor part is kept in memory.

Root sector:

The first sector of a physical device.  This contains information about the device: size of device in number of sectors;  locations, types and sizes of partitions; location and size of the bad-sector list.

Partition:

A GEMDOS "logical" device, such as C:.  There may be from one to four partitions per physical unit, and possibly more with an extension to the device partition table.

Partition header:

The root sector, FATs, and root directory of a GEMDOS partition.  It must be contiguous, with no intervening bad sectors.

Boot sector:

The first sector of a partition.  This gives GEMDOS information about the partition such as its size, the size of its FAT, and how large the root directory is.

FAT:

File Allocation Table.  This is where the clusters assigned to a given file in the GEMDOS filesystem are recorded, and also where bad sectors within a GEMDOS partition are marked.

Root directory:

The root (topmost) directory of a device.

The Bad-Sector List

The second and subsequent sectors of the media are reserved for the media-wide BSL.  The size of the BSL is based on the device size, and is fixed at formatting

time.  This BSL consists of 3-byte entries.  The first entry is spectial, and is described below.  The rest of the entries consist of 3-byte sector numbers which are bad.  This system has an upper limit of 8 Gbytes per device.  Entries in this list may straddle physical sectors, and the last sector in the list is zero-filled, since sector zero can never be bad on a working device.

The first 3-byte entry in the media BSL contains the number of sectors in the BSL, the number of bad sectors in the vendor BSl, and a checksum byte which causes the whole BSl, when added bytewise, to sum to A5 hex.  (If this criterion is not met, the whole BSL is assumed to be bad.)  The first N entries in the BSL are vendor bad sectors, where N is the second byte of hte first entry.  The remainder of the BSL is for user-bad sectors.  The user-bad list is cleared out when the device is reformatted, but is retained during partitioning:  the granularity of bad-sector mards in FATs is two sectors, and the user list records sectors.

The size of this list is set when the device is formatted: it does not grow.  The variable size is included to leave flexibility for larger BSL regions without paying the penalty on smaller devices.

This list is used to remember bad sectors on the media independent of the partitioning scheme.  The bad sectors marked here are also marked in the FAT of each GEMDOS partition, where appropriate.  If the partitions are changed, the new FATs will reflect the same bad sectors, relocated appropriately.