TUTORIAL.DOC

============

RUNNING FHDX

The FHDX disk contains the following files:

AUTO    a folder that contains the following program(s):

AHDI.PRG (version 3.01)

- FHDX 3.4 uses this to read/write hard disks.

FHDX.PRG    Factory Hard Disk Utility Program

FHDX.RSC    FHDX GEM resource file

TUTORIAL.DOC    this tutorial

README.DOC    a "must read" document

WINCAP    configuration file for default unit, default disk

type, hard disk controller test, daisy-chain test,

format, partition, and burn-in test.

Double-click on FHDX.PRG to run the hard disk  utility.

Move  the  mouse cursor to the "Disk" menu and click on

Select  Unit,  Select  Disk  Model,  Controller   Test,

Daisy-chain  Test,  Format  Disk,  Input  Defect  List,

Burn-in Test, or Ship.

October 2, 1989

- 2 -

SELECT UNIT

Click on the appropiate unit number, and then click  on

OK  to  select that unit, or CANCEL to abort.  The unit

number displayed on the screen will be  updated.   This

unit  number  will  be used throughout the FHDX session

until another unit  number  is  selected  using  SELECT

UNIT.

SELECT DISK MODEL

Click on the appropiate disk type.  If  further  selec-

tions  are given, click on the appropiate model number.

Click on OK to select that  disk  type,  or  CANCEL  to

abort.   The  disk type displayed on the screen will be

updated. This disk model will be  used  throughout  the

FHDX session until another disk model is selected using

SELECT DISK MODEL.

CONTROLLER TEST

When the test is finished, the screen will either  turn

red  if  the  disk failed the test, or stay green if it

passed.  Hit Return or click on the message box in  the

middle to get back to the main menu.

DAISY-CHAIN TEST

When the test is finished, the screen will either  turn

red  if  the  disk  failed the test (the message in the

middle of the screen will tell  you  whether  the  disk

failed  on  a  write error, a read error or a data com-

parison error), or stay green if it passed.  Hit Return

or  click  on the message box in the middle to get back

to the main menu.

October 2, 1989

- 3 -

FORMAT DISK

The format would begin as soon as you click  on  Format

Disk  on  the  menu.  Please note that the disk will be

partitioned after it is formatted.  The default  parti-

tion scheme for each disk type is specified in the WIN-

CAP file.

INPUT DEFECT LIST

(1)  If a defect list already exists, a  dialogue  box  will

come  up  on the screen and ask if you want to keep the

defect list that already exists.  Click on Yes to  keep

the  current  list.   Click on No to ignore the current

list.

(2)  A dialogue box will come up on the  screen  to  inquire

the  type  of  tester  used  to  generate  the  defects

recorded by the hard disk vendor.  Select  MFM  or  RLL

and then click on Start to begin inputting, or click on

Cancel to abort.

(3)  A form will come up on the screen with blanks for  Head

number,  Cylinder  number and Byte offset.  Fill in the

blanks with the  appropiate  number  from  the  defects

recorded  by  the hard disk vendor.  To move the cursor

through the entries, use the 'Tab' key or  the  up/down

arrow  keys.   Click on Next (or hit 'Return') to enter

the next defect, or click on Done when you are finished

inputting.

(4)  A dialogue box will come up on the  screen  to  inquire

what  to  do  next.   You  should click on 'Examine the

defect list' if you have just  finished  inputting  the

defects.   Click  on 'Save the defect list' to save the

defects and leave.  Click on 'Quit without  saving'  to

leave without saving the defects just entered.

(5)  If you have clicked on 'Examine  the  defect  list'  in

step  4,  a  window will come up displaying the defects

that you have just entered.  Examine each  entry  care-

fully.   If not all of the defects are displayed, click

on the up/down arrows or the scroll  bar  to  move  the

content in the window up or down.

(6)  If you find an incorrect entry, click on the entry.   A

form  will  come  up with the entered numbers.  Correct

the wrong number(s).  Click on OK to save the corrected

numbers, click on Delete to remove that entry, or click

on Cancel to ignore the correction(s).  Deleted entries

will be disabled in the defect list window.

October 2, 1989

- 4 -

(7)  After examining all entries thoroughly,  click  on  the

close box on the upper left hand corner of the window.

(8)  A dialogue box similar to the one in step 4  will  show

up.  Click on the action you want to take (see step 4).

BURN-IN TEST

(1)  The burn-in test begins as soon as  you  click  on  the

menu entry.

(2)  A dialogue box will come up on the screen informing the

progress  of the test.  To suspend the test, hit <ctrl-

s>.  To  resume  the  test  after  suspending  it,  hit

<ctrl-r>.  To quit the test, hit <ctrl-q>.

(3)  After you quit the test, the screen  will  either  turn

red  if  the  disk failed the test, or stay green if it

passed.  Hit 'Return' or click on the  message  box  in

the middle of the screen to exit.

(4)  A window will then come up displaying the  defects  (in

form of head number, cylinder number and sector number)

that have been logged on the drive.  If there are  more

defects  than  the  window  could  display at one time,

click on the up/down arrows or the scroll bar  to  move

the  content  in  the  window up or down.  Click on the

close box on the upper left hand corner of  the  window

or hit Return to get back to the main menu.

SHIP

An alert box will come up on the screen to  remind  you

to  turn  off  and  disconnect your hard disk after the

disk heads are parked.  You MUST switch off  the  power

of the disk once its heads have been parked.

October 2, 1989