Filename: 9600DATA.TXT

Version : 1.2b

Updated : 1/11/1991

----------------------------------------------------------------------

A Comparison of High Speed Modems

Compatibility/Features/Differences/Prices

Including: HST, V.32, V.32bis, V.42, V.42bis

By Mike Ehlert, SysOp: PACIFIC COAST MICRO BBS

USR HST/V.32bis (805) 494-9386

USR HST 14.4k   (805) 497-3456

CC SpeedModem   (805) 496-7320

FidoNet 1:206/2801

----------------------------------------------------------------------

TABLE OF CONTENTS

A High Speed Modem

Disclaimer

Standards

The CCITT

Half-Duplex Vs. Full Duplex

What is MNP, HST, V.32, V.42 ect.

What to look for: Throughput, Configurability, UARTS, Etc.

Some Questions to ask: If Considering a generic V.32/V.42.

Some Questions to ask: If Considering a FAX/Modem.

Modem Manufacturer Phone Numbers.

Modem Compatibility Listing.

Modem Price Comparisons.

Document Updates.

----------------------------------------------------------------------

A High Speed Modem

==================

Are you thinking of going to a High speed Modem?

2400 baud can  be  very  slow  for  transferring  large  files. Modems

advertised as 4800 are really only  2400 with data compression such as

MNP-5 or even just software data  compression. Don't be fooled. A 9600

or faster Modem will  make  a  tremendous  difference to your transfer

time. Most 9600 Baud users never want  to  go back to 2400. There is a

lot of  confusion  and  mis-information  about  9600  Baud  and faster

Modems. I wrote  this  article  to  help  clarify  and  point  out the

differences  between  the  different   Modems   available  for  MS-DOS

machines.

If price is no object, the best  Modem  to  get  for BBS use is the US

Robotics Dual Standard. It  is  a  combination  of  the HST 14400 Baud

Modem and a V.32/V.42 Modem all rolled into one.

If price is an issue, (like it is to most people) then you may want to

consider a Standard HST 14.4k, or  a  different  brand if that is also

too expensive. But if you do not buy a US Robotics Modem, you will not

be able to hook up to  a  Standard  USR  HST  14.4k, which is the most

common BBS Modem.

Other then USR, you could get a V.32 or V.32/V.42 compatible Modem for

about 25% less then a Standard HST,  or you could get a 9600 FAX/Modem

for less then half the cost  of  a  V.32  See the price comparisons at

the end.

Disclaimer

==========

This information was obtained from several reputable sources, but none

of it is guaranteed to be 100%  accurate. I am not responsible for any

incorrect information in this document,  nor  am I responsible for any

loss of profits to anyone  resulting  from  reading the information in

this documentation. Proceed at your own risk.

----------------------------------------------------------------------

Corrections and updates

=======================

Since  new  Modem  brands,  models,   and  new  prices  are  appearing

constantly, this documentation is  going  to  need  a lot of updating.

Also some of the Current Information may need corrections as well. You

are welcome to make changes to this  information, but please send me a

copy of your update so  that  I  can  add  your changes to the current

latest version I have. That way all the corrections that are made will

be added to the  latest  version,  which  will  also  be available for

download on my BBS.

----------------------------------------------------------------------

STANDARDS

=========

At one time Hayes was the  standard  in Modems. All other Modems tried

to be "Hayes Compatible". This is  still  true  with 1200 and 2400 bps

Modems, but  NOT  when  it  comes  to  9600  or  faster  Modems. Hayes

established  a  Standard  called  the  AT  command  set.  Other  Modem

manufacturers quickly adopted this  standard  and  the AT commands are

still used today (with a few commands added).

Well along came US Robotics, and  designed the HST, a proprietary 9600

Modem that quickly became  popular,  and  no  one  else was allowed to

duplicate. Hayes then also came out with a proprietary 9600 Modem, but

since it could not talk to the HST  at 9600 it never became popular in

the IBM world. Next USR improved  their  HST Modem, making it transfer

at 14400 bps without even using data compression. USR's HST became the

standard for MS-DOS compatible High Speed Modems.

Realizing that the industry  needed  a "non-proprietary" 9600 standard

that any manufacturer could  use,  the  CCITT  came  up  with the V.32

specification. It took  a  several  years  before  Modem manufacturers

started finding ways to implement this standard at a price competitive

to HST, but  a  few  manufacturers  started  producing them, including

Hayes. Next the CCITT realized  that  better error correction and data

compression methods existed, and thus came  up  with the V.42 and .V42

BIS standard. They are currently finalizing the V.32bis specification,

which will also run at 14,000 Baud in full duplex.

Now Modem Manufactures all seem to  be  jumping on the V.32 bandwagon,

even USR offers V.32/V.42 Modems and  allows  the HST's to be upgraded

to a "Dual-Standard" so it can be  both  HST and V.32/V.42 in one. The

new HSTs also have V.42bis and can  be upgraded to the new V.32bis DS.

The price difference between the  HST  and  the HST-DS is considerable

though, which is why most  BBS  Sysops  still  buy the HST without the

"DS" option.

The V.32/V.42 have become  the  standard  in  the Macintosh world, and

some MS-DOS BBS's are now running  V.32/V.42 instead of supporting the

HST, and some are supporting both types, using multiple lines.

Fax Machines have had their own  standards, CCITT V.27, V.27, and V.29

define the modulation scheme  used  for  Fax,  and  T.30 defines a Fax

Protocol. Fax machines nowadays  can  transfer  Faxs  at  4800 or 9600

Baud. Most Fax/Modems can only transfer  files  at 2400 Baud. There is

now one Fax/Modem which can also  transfer  files  at 9600 bps, and is

far less expensive then the V.32/V.42s,  plus  offer the benefits of a

Fax machine.

----------------------------------------------------------------------

The CCITT

=========

The International Consultative Committee  for  Telegraph and Telephone

(CCITT) set  the  V.32,  V.42,  and  V.42bis  Standards  in  9600  bps

communications. They have also set the  standards of 1200 and 2400 bps

in Europe.  Unfortunately  the  V.32  standard  is  very  expensive to

implement for the Modem manufacturers, and  the HST has already become

the industry standard in the IBM BBS  world. But since any company may

manufacture V.32/42 Modems without a licence or royalty, there is more

price-competition going  on  with  these  than  with  the  HST,  which

currently has no true competition.

----------------------------------------------------------------------

Half Duplex Vs. Full Duplex

===========================

Half-Duplex is when data is  transmitted  in  one direction at a time.

Full-Duplex is both directions  at  once.  2400  baud Modems are Full-

Duplex. The HST Modem uses an  improved  form of Half-Duplex, in which

one direction is transferring at high  speed while the other direction

is sending ACK/NAK signals at a slower speed. A similar method is used

on the SpeedModem.

V.32 Modems use Full  duplex,  which  in  most  cases does not improve

performance when used  on  a  BBS.  There  is  one  way  to  take full

advantage of a Full-Duplex Modem:  By  using a protocol called BiModem

to upload and download at the same time at 9600 bps in each direction!

Unfortunately BiModem has  not  become  very  popular.  Most BBS's and

users still use one  direction  transfer  protocols  such as ZModem or

YModem-G.

----------------------------------------------------------------------

What is MNP, HST, V.32, V.42?

=============================

MNP

MNP (Microcomm Network  Protocol)  levels  1-4  are  methods  of error

correction in which the two  Modems  connected verify the integrity of

the  data  transmitted.  Error  correction  is  required  for  several

streaming protocols such as  YModem-G  in  which  the protocol sends a

constant stream of data and lets  the  Modems do the error correction.

This requires a clean,  noise  free  line  as streaming protocols will

abort if line noise interferes too much.  These MNP Levels are used in

almost all High Speed Modems Made  today,  But just because two Modems

both have MNP it does  not  ensure  that  they  will both talk to each

other (at over 2400 baud).

MNP Level 5 is for data compression.  Since all BBS's have their files

archived in ZIP, LZH, GIF or some such compressed format (try to ZIP a

LZH file), MNP5 can actually  increase  the  overhead by attempting to

compress the file further. Therefore BBS's  leave MNP5 turned off, and

so should the BBS callers.

----------------------------------------------------------------------

HST

HST stands for "High Speed Technology"  - a method of communicating at

high speeds which was  developed  by  US  Robotics  in  1984. HST is a

proprietary method so currently only  US  Robotics  is allowed to make

HST compatible Modems. The original HST  Modems were 9600 bps by using

a 9200 bps channel in one  direction  and  a 450 bps "back channel" in

the other to send ACK/NAK signals  for  confirmation  of data - a half

duplex mode  -  often  referred  to  as  "ping-ponging".  In  1989 USR

improved the HST to  run  at  14400  bps  by  further optimizing their

proprietary method. All HST's sold now  include the 144k speed as well

as the original 9600 mode. Because  the  HST  14.4k is a raw speed, It

allows the HST to transfer pre compressed files such as ZIPs at faster

speeds then any other 9600 Modem: over 1700 Characters per second.

The HST has become the "BBS-Standard" in high-Speed Modems. More BBS's

use the HST then any other type of Modem, including the V.32.

As of January 1st 1991  all  HST  Modems also include the V.42/V.42bis

error correction and data compression (not to be confused with V.32).

The HST's can also be upgraded to  the HST "Dual Standard" allowing it

to be V.32bis compatible. The  price  is  significant for this upgrade

however, and in very few cases is any speed gained.

----------------------------------------------------------------------

V.32

V.32 is a 9600 Baud standard that was established by the CCITT back in

1984.  V.32 is full  duplex  (9600  bps  in  both directions at once).

Normally this would be impossible, but  using a technique called "echo

cancellation", one Modem can filter its own tones from the phone line,

enabling it to pick out  the  signals  from  the other Modem. However,

echo cancellation requires  that  high  speed  Modems include built-in

digital signal processor (DSP) chips,  which  is  the main reason V.32

and V.42 costs so much. V.32 also includes a fallback to 4800, if line

noise becomes a problem at  9600.  The  V.32 standard also provides an

optional  error-reduction  scheme,  called  "trellis-coded  modulation

(TCM). TCM allows 9600 Modems to  check for transmission errors with a

redundancy bit, which results  in  fewer  errors  on noisy lines. Most

V.32's include this option, but some do not.

----------------------------------------------------------------------

V.32bis

It was first realized in 1989 that  it  is feasible to push V.32 up to

14,400 baud, the same speed as  the  HST  now runs. This requires even

better echo cancellers and an overall improvement in receiver quality,

so it will be more costly  to  produce  the  V.32. V.32bis has not yet

officially been finalized CCITT, but as of January 1st US Robotics has

started shipping a new  version  of  their  Dual  Standard Modem which

includes V.32bis in an early but  functional form. USR plans to update

their DS ROMS when the  CCITT  actually  release the completed V.32bis

specification, scheduled for mid 1991.  It's expected that other Modem

manufacturers will not be able  to  offer the V.32bis technology until

the 4th quarter of 1991.

----------------------------------------------------------------------

V.42

V.42 is nothing to  do  with  actual  Modem  speed,  but how the error

correction and data  compression  schemes  interface.  When  the CCITT

approved V.42 in 1988,  they  decided  to  include two different error

correction and data compression schemes.  The  first one is MNP,(short

for Microcom Networking Protocol). MNP classes  2, 3, & 4 handle error

correction. Note that MNP-5  was  not  included  in  the V.42 standard

because it is only used for data compression. MNP was made an official

standard due to the large number of  Modems that currently use it. The

second method is the  preferred  method,  known  as LAP-M (Link Access

Protocol-Modem). A Modem with only MNP is called "V.42 Compatible" and

one with LAP-M its  known  as  "V.42  Compliant".  If a V.42 Compliant

connected to a V.42 compatible, it  would  first try to handshake with

LAP-M, and after it realized that the other Modem is not compliant, it

would try the MNP Method, and they connect.

Most V.42 Modems are fully V.32  compatible,  but  they do NOT have to

be, according to CCITT  guidelines.  So  when  purchasing a V.42, make

sure that it is really a  V.32  Modem  that  is also V.42 compliant or

compatible. There are now some 2400 baud Modems which are also V.42bis

compliant, so since the  LAP-M  gives  4:1  data  compression they are

advertised as "9600 throughput",  which  really  can be misleading and

confusing.

----------------------------------------------------------------------

V.42bis

V.42bis is a new CCITT standard for data compression techniques, which

was approved in late 1989.  To  support  V.42bis, a Modem MUST support

both LAP-M and MNP-5, unlike the standard  V.42 in which LAP-M is only

an option. V.42 provides a maximum  data  compression of 4:1, giving a

9600 bps Modem a  throughput  up  to  38400  bps.  MNP only offers 2:1

Compression. Like MNP-5, LAP-M and V.42bis  will not be effective when

transferring compressed files from  BBS's,  such  as  ZIP files. Since

V.42 is implemented in  firmware,  many  V.42  compliant Modems can be

upgraded to V.42bis with a new ROM.

----------------------------------------------------------------------

SpeedModem

A less expensive method  of  9600  bps  communication  can be achieved

without using the CCITTs method  of  "echo  cancellation", so that the

digital signal processor is  not  needed,  making  the  cost much less

expensive. This  method  is  used  on  the  Compucom  SpeedModem.  The

SpeedModem also uses Dynamic Impedance Stabilization (DIS) to increase

the reliability of the  telephone  interfaces signal-to-noise ratio of

the telephone line by increasing the  clarity and power of the signal,

and automatically compensating for  impedance  variations on the phone

line. This  reduces  the  probability  of  line  noise,  allowing  the

Protocols to transfer files with  less  overhead  It also has a faster

fallback rate of 7200  bps  if  too  much  line  noise exists for 9600

communications. This makes the SpeedModem stand up to worse line noise

conditions at a faster rate  then  V.32  can handle. The SpeedModem is

also a 9600 bps Group III Facsimile,  so with this Modem, you can send

document and scanned images as  a  Fax,  as  well  as receive Faxs. It

supports BitFAX by Bitcom and any  other  3rd party FAX software which

uses the standard CCITT AT-FAX command  set. A Data Compression called

CSP (CompuCom Speed Protocol) is used to  yield a compression of up to

4:1, giving a throughput of up  to  38400 bps. This however, like MNP5

will not be effective when  transferring  compressed files from BBS's,

such as ZIP files.

DIS and CPS are proprietary methods  owned by CompuCom, Sunnyvale, CA.

Because this Modem is inexpensive  more  people  can afford to buy it,

and since it is  both  a  FAX  and  a  Modem  in  one,  it should gain

popularity quickly. It is still a  new  product, but hundreds of BBS's

are showing support for it.

----------------------------------------------------------------------

What Next?

By mid 1991 the V.32bis standard will  be finalized, and by that years

end several 14,400 Bps full duplex Modems will be on the market. There

is already talk of the CCITT releasing another standard in 1992, which

will be for 192,000  bps.  Since  these  Modems  are  expected to cost

considerably more then the V.32s currently on the market, they will be

out of the price range of most BBS users.

----------------------------------------------------------------------

WHAT TO LOOK FOR

THROUGHPUT:

==========

You'll see ads claiming that you  can  attain 19,200 bps or 38,400 bps

with a 9600 Baud Modem - this  is  true  only if you are using MNP5 or

other form of data compression on  an  uncompressed data or text file.

Any 9600 Baud Modem using compression  will transfer at that effective

rate. But for BBS use,  where  files  are  already compressed (such as

ZIPs,  LZHs,  GIFs)  MNP5  will   actually  slow  down  throughput  by

attempting to compress a  file  that  is already compressed. Generally

when using a Modem for BBS transfers,  it's the RAW speed of the Modem

and not the throughput which is important.

Figures of around 1700  cps  by  the  HST  Modems are attained through

proprietary methods not available on other Modems.

CONFIGURABILITY:

================

In High Speed Modems, there is a lot  more to configure than on a 2400

Baud Modem. Be sure your  Modem  comes  with NRAM (Non-volatile Random

Access Memory) that can  store  your  Modem configuration, making long

init strings unnecessary. Also many  Modems  have  dip switches to set

the default configuration, which also simplifies things. If you buy an

Internal Modem, be sure its fully Configurable as far as selecting the

COM port and  IRQ  (interrupt  request)  setting.  Some  Modems can be

addressed from COM1: up to COM8:

UARTS

=====

Internal Modems have a serial port  built  in, and External Modems use

an existing Serial Port in the computer. A serial port has a UART chip

to buffer and control the  Input/Output  (I/O).  The XT usually has an

8450 UART which will handle up  to  9600  Baud. The AT usually has the

16450. If you have an external Modem you should replace your UART with

the 16550 (NS16550AN).

INTERNAL or EXTERNAL

====================

MS-DOS users can choose internal  Modems,  that  plug into a computers

slot, or external, which plugs  into  an  existing  serial port with a

serial cable. Internals  are  usually  less  expensive,  take  up less

space, have the correct UART  already  installed (see below) and don't

need a cable. But the Externals usually have a lot of little lights on

it that you can stare at, and  its  easier to move to another machine.

So which ever you prefer (or is available in some cases).

If Considering a USRobotics Modem

=================================

You only need to worry about  whether  you  want the HST (which can be

upgraded later), or the Dual Standard which is the same Modem with the

V.32bis option included.

If Considering a V.32/V.42 Modem

================================

Some questions to ask:

Can you afford the USR "DS" instead?

Is it both V.32 and V.42 compatible?

Is it V.42 compatible, V.42 compliant, or V.42bis?

If it's not V.42bis or compliant, can it be upgraded?

Do Local/National BBS's support it?

Would you rather wait a year for a V.32bis?

What is the warranty? 5 years?

If Considering a FAX/Modem

==========================

some questions to ask:

Is it Send and Receive Fax?

Does it support 3rd party FAX software?

Is it 4800 or 9600 bps?

Does it support MNP?

Does it support other compression?

Do Local/National BBS's support it?

What is the Warranty? 5 Years?

----------------------------------------------------------------------

Modem Compatibility Listing:

============================

These Modems will usually be downwardly  compatible with 2400 Baud and

slower Modems, which  means  they  are  also  CITTY  V.22bis  and Bell

103/212A compatible. Most have an  auto-fallback mode that will detect

the highest negotiable Baud rate, which can be either through hardware

or software configuration. If the Modem  is V.42 capable, it will fall

back to MNP if the  other  Modem  is  not  LAP-M   capable, but is MNP

capable.   Modems   supporting    MNP    will    connect   with   data

compression/error correction with other MNP Modems at the highest Baud

rate negotiable between the two. The speeds listed here are the actual

raw speed, not possible  throughput  that  can  be achieved using data

compression.

Listed in alphabetical order by brand name -

Generic V.32 only

Made by a number of companies  using the CCITT method of communicating

at 9600 bps. These are expected to become outdated as V.42 is added to

most V.32 Modems

- Talks to other brand V.32 Modems at 9600 Baud.

Generic V.42 only

If a Modem is V.42 or  V.42bis  only,  and  not V.32, then it can only

talk to other V.42 Modems  at  2400,  (with 9600 throughput using data

compression) This  is  not  considered  a  9600  Modem  and  should be

avoided.

- Talks to other brand V.42 Modems at 2400 Baud with LAP-M.

Generic V.32/V.42 (and V.42bis)

These Modems  follow  BOTH  the  CCITT  V.32  and  V.42  standards for

communicating at 9600 bps.

- Talks to other brand V.32 Modems at 9600 Baud.

- Talks to other brand V.42 Modems at Top Supported Baud.

(Some Generic  Brands  include:  Anderson  Jacobson,  Codex,  Computer

Periphels,  Digicom,  E-Tech,  Farallon,  Fastcomm,  General  Datacom,

Intel,  Magic,  Mastercom,   Microcom,   Multi-tech,   NEC,  Practical

Peripherals, Prometheus,  Radcal  Vadic,  Shiva,  Telebit, Telenetics,

USD, and many others just  appearing.  Some  of  these offer LAP-M and

V.42bis, and some do not).

Hayes V-Series:

The early Hayes V-Series uses a proprietary method of communicating at

9600 bps.

- Talks other Hayes V-Series Modems at 9600 Baud.

Hayes V-Series V.42:

The Hayes V-Series V.42 uses  the  proprietary method of communicating

at 9600 Baud but will incorporate  the V.42 data compression and error

checking

-   Talks   to   other    V.42    Modems    at    2400   with   error

correction/compression.

-    Talks   to   other    MNP    Modems    at    2400   using   error

correction/compression.

- Talks to other Hayes V-Series at 9600.

Hayes ULTRA:

Uses CCITT V.32 and V.42bis method for communicating at 9600 Baud.

Has MNP5 abilities.

Has LAP-M abilities

- Talks to Other V.32 Modems at 9600 Baud

- Talks to other V.42 Modems at up to 9600 Baud

- Talks to other Hayes V-Series at 9600

Telebit Trailblazer:

Uses proprietary PEP method of communicating at 9600 Baud.

Has MNP5 abilities.

- Talks to other Telebit Trailblazers at 9600.

-  Talks   to   other   MNP   Modems   at   2400   baud   with  error

correction/compression

USR Courier V.32:

Uses CCITT V.32 and V.42 method of communicating at 9600 Baud.

Offers MNP5 abilities.

- Talks to Other V.32 Modems at 9600 Baud.

Talks to Other V.42 Modems at up to 9600 baud

USR Courier HST: (old version)

Uses the USR proprietary HST method of communicating at 9600 Baud.

Has MNP5 abilities.

- Talks to HST type Modems at up to 9600 Baud.

USR Courier HST 14.4:

Uses the USR proprietary HST method of communicating at 9600 Baud.

Has MNP5 abilities.

- Talks to HST DS's at 14,400 Baud

- Talks to HST 14.4's at 14,400 Baud

- Talks to HST's at 9600 Baud

USR Courier HST DS:

The Dual Standard  incorporates  both  the  proprietary  HST method of

communicating at 14400 baud  and  the  CCITT  V.32  &  v.42 methods of

communicating at 9600 Baud. (and V.32bis after 1/1/1991)

Has MNP5 abilities.

- Talks to Other V.32 Modems at 9600 Baud.

- Talks to Other V.32bis Modems 14,400 Baud.

- Talks to Other DS's at 14,400 Baud

- Talks to 14.4 HST's at 14,400 Baud

- Talks to old 9600 HST's at 9600 Baud.

COMPUcom SpeedModem/Fax:

Uses the Proprietary DIS at 9600 Baud.

Has 7200 and 4800 Baud Fallback

Has CCITT FAX V.27ter & V.29

Has MNP5 abilities.

Has CSP abilities.

Talks to Group III Faxs at 9600.

- Talks to SpeedModems at 9600.

- Talks to other MNP Modems at 2400 with error

correction/compression

----------------------------------------------------------------------

Manufacturer           Toll Free       Charge Call     Support BBS

============          ==============  ==============  ==============

Anchor                (800) 541-2318

Anderson Jacobson     (800) 438-8520  (408) 435-8520

Codex                                 (508) 261-4000

Compucom              (800) 228-6648  (408) 732-4500  (408) 738-4990

Computer Peripherals  (800) 854-7600  (805) 499-5751  (805) 499-9646

Digicom               (800) 574-2730

E-Tech                                (408) 982-0270

Farallon                              (415) 596-9100

Fastcomm              (800) 521-2496  (703) 620 3900

General Datacom       (203) 574-1118

Hayes                 (800) 241-9625  (404) 441-1617  (800) 874-2937

Intel                 (800) 538-3373                  (503) 645-6275

Magic Modems          (800) 622-3475

Mastercom                             (213) 834-6666

Microcomm             (800) 822-8224  (617) 551-1000

Multi-Tech            (800) 328-9717

NEC                   (800) 222-4632  (408) 433-1250

Practical Peripherals (800) 442-4774  (818) 706-0333

Prometheus            (800) 477-3473  (503) 624-0571

Radcal Vadic          (800) 482-3427  (408) 432-8008

Radcal Milgo          (800) 327-7909  (305) 846-1601

Shiva                 (800) 458-3550  (617) 864-8500

Telebit               (800) 835-3248  (408) 734-4333

Telenetics            (800) 822-4267  (714) 779-2766

USD                   (800) 631-4869  (205) 430-8000

USRobotics            (800) DIAL-USR  (708) 982-5001  (708) 982-5092

----------------------------------------------------------------------

Price Comparisons

=================

No Prices have been stated  above  this  section, because prices often

change, and I want to keep  all  the  prices together so they could be

quickly updated as needed. There are  3  types of prices I will cover.

First is The Retail price, second  is  the  Sysop price, which is only

available if you run a BBS.

Many Modem manufacturers offer BBS  Sysop  special prices directly, at

about 50% off the retail price. First  the  BBS has to be verified, so

the Modem manufacturer knows that is a  real BBS that has been running

for at least 6 Months and has  a minimum number of users, etc. Usually

the manufacturer will call the BBS once,  or twice a few week apart to

be sure. Verification usually take  3-4  weeks.  Then the Sysop has to

agree to use the Modem on the BBS for some set amount of time.

-----------------------------------------------------------------

Generic Brands         Retail          Sysop

V.32 Only    EXT       $650-$1,000    ????

V.32/V.42    EXT       $700-$1,000    ????

V.32/V.42bis EXT       $700-$1,200    $339+

Software Included: Varies

Warranty: Varies 1-5 years

-----------------------------------------------------------------

Hayes                    Retail      Sysop

Ultra V.32/V.42          $1,199      $599

V-Series V.42 (not V.32)   $999      $499

V-Series 9600              $799      $399

Software Included: None

Warranty: 2 years

-----------------------------------------------------------------

Compucom                 Retail      Sysop

SpeedModem/Fax INT         $279      $169

Software included: BitFax

Warranty: 5 years

-----------------------------------------------------------------

Intel                    Retail      Sysop

9600EX                     $799      $399

Software included: None

Warranty: 5 years

-----------------------------------------------------------------

Practical Peripherals    Retail      Sysop

V.32/V.42  EXT             $699      $339

V.32/V.42  INT              N/A      N/A

Software Included: None

Warranty: 5 Years

Note: Due to many reports of Incompatibilities/Problems from owners of

these new P.P. Modems, I would  not currently recommend them, although

they are currently the lowest priced V.32 I have seen.

------------------------------------------------------------------

Telebit                   Retail       Sysop

T1000                      ???        ???

T1500 V.32                 ???        ???

TrailBlazer INT            ???        ???

Trailblazer EXT            ???        ???

T2500 V.32/V.42            ???        ???

Software Included:None

Warranty: 5 years

------------------------------------------------------------------

USRobotics                Retail       SysOp

Courier Dual Standard E   $1,595       $699

Courier Dual Standard I   $1,395       $649

Courier V.32/V.42 EXT     $1,099       $599

Courier V.32/V.42 INT       $949       $577

Courier HST 14.4  EXT       $995       $449

Courier HST 14.4  INT       $895       $405

DS Upgrade Kit for HST EXT  $600       $300

DS Upgrade Kit for HST INT  $500       $250

Software included: None

Warranty: 2 Years

------------------------------------------------------------------

Document updates:

Date       Release    Comments

----       -------    --------

10/30/90   1.0        Init Release. Should have run a spell checker!

11/08/90   1.1        Spelling fixes, V.32bis update, Compucom update

01/10/91   1.2        USR V.32bis Update, other minor corrections

01/11/91   1.2b       SpeedModem Corrections submitted to Remco

Treffkorn for feedback (not for public release)

** End-of-file **