This little patch fixes two problems in the Falcon BIOS (TOS 4.00-4.04)

(1) Sound is not initialized properly.

(2) Bconmap() doesn't work.

To use it, copy FPATCH2.PRG to your AUTO folder.

This patch supersedes FPATCH1.PRG. The former worked by hooking into

the BIOS trap and intercepting Bconxxx calls. This patch fixes the

Bconmap problem directly (by setting the Bconmap table size to its

correct value) and hence requires no resident code. It also has

the benefit of allowing the Falcon LAN port to be accessed via

the BIOS.